|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 102 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Other Species/ Scientific Review |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  | X |  |  |  |  |
|  | FINAL ACTION |  |  |  |  |  | X |  |  |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | Page 25 and 29 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | Modify the 2007 PMO, page 25, Section 6, Laboratory Techniques, item 6b. 6b. Goat Milk: Direct Microscopic Somatic Cell Count or Electronic Somatic Cell Count may be used for screening raw goat milk samples, to indicate a range of somatic cell levels, as long as the somatic cell standard for goat milk remains 1,000,000/mL 1,500,000/mL. Screening for official purposes must be conducted by an analyst (s) certified for that procedure. Only the Pyronine Y-Methyl Green stain or “New York modification” Single Strip Direct Microscopic Somatic Cell Count test procedures shall be used to confirm the level of somatic cells in goat milk by certified analysts. Modify the 2007 PMO, page 29, footnote, Section 7, Table 1. * Goat Milk 1,000,000 per mL 1,500,000 per mL |  |  |  |  |  |  |  |  |  |  |

| 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  |  |  | Proposal #: |  | 104 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Committee: |  | Scientific Review |  |  |
|  |  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |  |
| COUNCIL ACTION |  |  |  |  |  |  |  |  |  | X |  |  |  |
| FINAL ACTION |  |  |  |  |  |  |  |  |  | X |  |  |  |
|  | C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Changes to be made on page(s): |  |  | 26 |  |  |  | of the (X - one of the following): |  |  |  |  |
|  |  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |  |
|  |  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |  |
|  |  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |  |
|  |  | Section 7 Standards for Grade “A” Milk and Milk Products Second paragraph: No process or manipulation other than pasteurization, ultra-pasteurization, or aseptic processing methods integral therewith; and appropriate refrigeration with or without the use of CO2 shall be applied to milk and milk products for the purpose of removing or deactivating microorganisms, provided that filtration and or bactofugation processes are preformed in the milk plant in which the milk or milk product is pasteurized, ultra-pasteurized or aseptically processed. Suspend the requirements of the PMO section 7 allowing only for refrigeration for the purpose of a study/pilot to be conducted following the requirements of FDA and this Ordinance to examine the benefits of using Carbon Dioxide as a processing aid in raw milk during transportation. This study/pilot will involve the incorporation of Carbon Dioxide into raw milk at the time of loading onto transport for shipment, removal of Carbon Dioxide at final location prior to pasteurization, and be allowed to test market the product A study/Pilot to be permitted on this issue. . 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS; Proposal #:; 110. Committee:. No Action; Passed as Submitted; Passed as Amended. COUNCIL ACTION; X Substitute Solution. FINAL ACTION; X. C. Proposed Solution. Changes to be made on page(s):; 52; of the (X – one of the following):. X; 2007 PMO; 2007 EML. 2007 MMSR; 2400 Forms. 2007 Procedures; 2007 Constitution and Bylaws. 3. All farm bulk milk tanks manufactured after January 1, 2000 shall be equipped with an approved temperature-recording device. a. The temperature-recording device shall be operated continuously and be maintained in a properly functioning manner. Circular charts shall not overlap. Electronic records that comply with the applicable provisions of Appendix H. IV and V, with or without hard copy, may be used in place of temperature-recording records. b. The temperature-recording device shall be verified every six (6) months and documented in a manner acceptable to the Regulatory Agency using an accurate (+/- 1oC (2oF)) thermometer that has been calibrated by a traceable standard thermometer, within the past six (6) months, with the results and date recorded and the thermometer being properly identified, or by using a traceable standard thermometer that has been calibrated within the last year c. Temperature-recording charts shall be maintained on the premises for a period of a minimum of six (6) months and are available for review by the Regulatory Agency. Except that, the electronic storage of required temperature records, with or without hard copy, shall be acceptable, provided the computer and computer generated temperature records are readily available for review by the Regulatory Agency d. The recording thermometer should be installed in an area convenient to the milk storage tank and acceptable to the Regulatory Agency. e. The recording thermometer sensor shall be located to permit the registering of the temperature of the contents when the tank contains no more than ten percent (10%) of its calibrated capacity. f. The recording thermometer shall comply with the current technical specifications for tank recording thermometers. g. A recording thermometer and/or any other device that meets the intent of these Administrative Procedures and technical specifications and is acceptable to the Regulatory Agency can be used to monitor/record the bulk tank temperature. h. The temperature-recording charts shall properly identify the producer, date installed, tank or silo identification (if more than one), and signature or initials of the person removing installing the chart. PMO, section 7, item 18r, page 52 3. All farm bulk milk tanks manufactured after January 1, 2000 shall be equipped with an approved temperature-recording device. e. The temperature-recording device sensor shall be located to permit the registering of the temperature of the contents when the tank contains no more than ten twenty percent (10 20%) of its calibrated capacity. h. The temperature-recording records shall properly identify the producer, date installed, tank or silo identification, if more than one (1), and signature or initials of the person removing installing the record.. |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 114 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  |  |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 74 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2007 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | 10. Pasteurized milk and milk products, except those that are to be concentrated in membrane processing systems, are not strained or filtered, except through a perforated metal strainer. Provided, that pasteurized milk and milk products that are concentrated (condensed) in membrane processing systems may be filtered provided that a single service in-line filter that is sanitized after assembly, may be allowed, if it is a part of the membrane processing system. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 115 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Tech. |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 76, 87, 220, 246, 247, 252-256, |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | PMO, section 7, Item 15p. (B) 1.b. (2), Page 76 2) Both valves, and valve seats in the case of single-bodied double seat valves, are position detectable and capable of providing an electronic signal when not properly seated in the blocked position. (Refer to Appendix H., I., Position Detection Devices) for FDDs and Valve Seats in HTST, HHST, Aseptic and Other Process Systems. pmo, section 7, item 16p. (B) 2.b. (1), Page 87 1) The forward-flow of milk or milk product below the minimum pasteurization temperature shall be prevented by requiring the motive pumps(s) to be de-energized when the milk or milk product is below the pasteurization temperature and the valve is not in the fully diverted position; or by any other equally satisfactory means. For the detection of the FDD and valve seat positions, refer to Appendix H. Pasteurization Equipment and Procedures and Other Equipment, I. HTST Pasteurization, Position Detection Devices for FDDs and Valve Seats in HTST, HHST, Aseptic and Other Process Systems. pmo, appendix h, Page 218 Position Detection DEVICES Where the position detectability of FDDs and valve seats is required this may be accomplished by mechanical or electronic means, such as mechanical limit switches (micro-switches) or electronic proximity switches. These switches shall be capable of providing an electrical signal when the valve seat is in the fully closed position, provided further that the position detection capability is fully testable. Position detection devices (PDDs) shall be repeatable and capable of detecting valve seat movement of less than 1/16 inch (1.59 mm) 1/8 inch (3.18 mm) at all times. MAGNETIC FLOW METER BASED TIMING SYSTEM FOR HTST PASTEURIZERS pmo, appendix h, Page 246 GLOSSARY Input: Electrical signals applied to the computer and used by the computer to make logical decisions on whether or not to activate one or more outputs. Input consists of data from temperature and pressure instruments, liquid level controls, micro-switches position detecting devices (PDDs), and operator-controlled panel switches. pmo, appendix h, Page 247 Output: Electrical signals from the computer that turn on or off valves, motors, lights, horns, and other devices being controlled by the computer. Outputs may also consist of messages and data to the operator. Position Detecting Device (PDD): Mechanical limit switches (micro-switches) or electronic proximity switches capable of providing an electrical signal. pmo, appendix h, Page 252 Revise diagrams legend: MS = Microswitch” with PDD = Position Detecting Device pmo, appendix h, Pages 253-256 Revise diagrams: MS with PDD pmo, appendix h, Page 257 VII. CRITERIA FOR STEAM-BLOCK TYPE FDD SYSTEMS 3. The Primary Divert Valve and other critical valves shall be position detectable and fail-safe and be alarmed to provide protection when needed. Note: For the detection of the FDD and valve seat positions, refer to Appendix H., I., Position Detection Devices. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 117 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  |  |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 78 |  |  |  | of the (X – one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2007 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | 2. Except as permitted in Item 16p, there shall be no physical connection between unpasteurized products, dairy, non-dairy, or water, and pasteurized milk or milk products. Pasteurized non-dairy products or water not completely separated from pasteurized milk and milk products, shall be pasteurized in properly designed and operated equipment at times and temperatures which meet at least the minimum times and temperatures provided for in Definition FF. or Iin the case of water, shall meet at least the minimum times and temperatures provided for in Definition FF in equipment that may not meet Item16p or has have undergone an equivalent process found acceptable by FDA and the Regulatory Agency or shall have undergone a hazard evaluation and safety assessment of the specific water supply and application involved and has undergone an additional treatment to destroy or remove bacterial acceptable to the Regulatory Agency, in consultation with FDA, to ensure the water will not compromise the safety of the milk or milk product10. Supporting information shall be submitted to and approved by the Regulatory Agency. The supporting information may include, but is not limited to the following: |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 119 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | SRC & TERC |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 78 and 260 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | PMO Page 78 Item 15p (B) 2. Except as permitted in Item 16p, there shall be no physical connection between unpasteurized products, dairy, non-dairy, or water, and pasteurized milk or milk products. Pasteurized non-dairy products or water not completely separated from pasteurized milk and milk products, shall be pasteurized at times and temperatures which meet at least the minimum times and temperatures provided for in Definition FF or in the case of water, meet the requirements found in Appendix H Section IX, or shall have undergone an equivalent process found acceptable by FDA and the Regulatory Agency or shall have undergone a hazard evaluation and safety assessment of the specific water supply and application involved and has undergone an additional treatment to destroy or remove bacterial acceptable to the Regulatory Agency, in consultation with FDA, to ensure the water will not compromise the safety of the milk or milk product10. ## PMO Page 260 Appendix H Create a new Section IX at the end of the current Appendix H IX. Accepted Processes for the Creation of Pasteurized Equivalent Water ULTRAVIOLET LIGHT(UV) DISINFECTION OF WATER BACKGROUND UV light between 200-400 nanometers is well known for inactivating pathogenic microorganisms in water via several mechanisms, including the formation of DNA bonds (dimers) that inhibit reproduction and infectivity. Different microbes have different responses to specific wavelengths which also can account for differences in overall dose requirements. Some microbes can use their own enzymes and mechanisms, or take advantage of host cell enzymes to repair the damaged DNA, requiring higher doses of UV to cause irrevocable damage and effective pasteurization-level disinfection. Three critical factors determine a UV unit’s ability to reliably achieve the necessary dose at any point in time: The tranmissivity of the water to UV, the performance of the lamps, the hydraulics and rate of the flow in the disinfection chamber. Color, turbidity, particles and organic impurities can interfere with the transmission of ultraviolet UV energy and lower the disinfection efficiency below levels required to insure destruction of pathogenic organisms. Similarly, lamps can age unevenly and water can foul the protective sleeves and prevent light from reaching some pathogens. Hydraulic patterns or flow that is too high or too low can cause uneven distribution of the dose and leave some areas without adequate disinfection. Other important factors include the geometric configuration of the reactor, the power, wavelength and physical arrangement of the UV lamps, and the UV path length. Longer path lengths provide more opportunities for UV photon-microbe interaction and inactivation. . Ultraviolet UV lamps treat water instantaneously while it is flowing through the disinfection chamber but do not provide residual bactericidal action. Using UV for pasteurized equivalent water is not a substitute for appropriate maintenance, periodic flushing and sanitizing of the water distribution system inside the plant. CRITERIA The following is a list of criteria that is required to accept water treated with ultraviolet UV light to be considered equivalent to pasteurized water: - Ultraviolet UV light shall be applied so that all particles of water the entire volume of water receives at least the following dose when used as pasteurized water. - Low pressure UV at 2,537 Angstrom at 186,000 microwatt-seconds per square centimeter or a 4 log adenovirus equivalent. - Medium Pressure UV at 120,000 microwatt-seconds per square centimeter or a 4 log adenovirus equivalent - A fFlow or time delay mechanism shall be provided so that all water moving past the flow stop or divert valve receives the minimum dose listed required above. - The unit shall be designed to permit the frequent cleaning of the system without disassembly of the unit and shall be cleaned often enough to ensure that the system will provide the required dose at all times. - An automatic flow control valve, accurate within the expected pressure range, shall be installed to restrict flow to the maximum design flow of the treatment unit so that all particles receive the minimum dose listed above. - An accurately calibrated ultraviolet UV intensity sensor, properly filtered to restrict its sensitivity to the 2,500-2,800 Angstrom germicidal spectrum, shall measure the UV energy from the lamps. - There shall either be one sensor for each UV lamp.: - O one sensor for each UV lamp or - The lamps shall be tested every 6 months and the lamp with the lowest output shall be placed in front of the sensor - 7. The light shall adjust based on water quality measured with a real time UVT analyzer to assure that the dose is always calculated accurately and provided reliably., unless data over a year (four seasons) shows that the UVT does not vary by more than 2%. - A flow diversion valve or automatic shut-off valve shall be installed which will permit flow into the pasteurized product lines only when at least the minimum required ultraviolet UV dosage is applied. When power is not being supplied to the unit, the valve should be in a closed (fail safe) position which prevents the flow of water into the pasteurized product lines. - The materials of construction shall not impart toxic materials into the water either as a result of the presence of toxic constituents in materials of construction or as a result of physical or chemical changes resulting from exposure to ultraviolet UV energy. - The unit shall be designed to protect the operator against electrical shock and UV light exposure. - 10. The unit shall record the operating parameters (flow, UVT and dose) on a real time basis. These records shall be accessible to the Regulatory Agency for inspection. Electronically generated records, if used, shall meet the criteria specified in Appendix H. V. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 122 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Scientific Review |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  | X |  |  |  |  |
|  | FINAL ACTION |  |  |  |  |  | X |  |  |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 7 and 81 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | ADMINISTRATIVE PROCEDURES *If the fat content of the milk product is 10 percent (10%) or greater, or a total solids of 18% or greater, or if it contains added sweeteners, or is concentrated (condensed), the specified temperature shall be increased by 3ºC (5ºF). |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 123 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Tech. |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  | X |  |  |  |  |
|  | FINAL ACTION |  |  |  |  |  | X |  |  |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 82 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | Make the following changes to the 2007 PMO: Strike out text to be deleted and underline text to be added. section 7. standards for grade “A” milk and milk products staNdards for grade “a’ pasteurized, ultr-pasteurized and aseptically processed milk and milk products item 16p. pasteurization and aseptic processing administrative procedures Page 82 3. All milk and milk products shall be pasteurized, prior to the entrance into ro, uf, evaporator or condensing equipment, and shall be performed in the milk plant where the processing is done, except that: …. b. If the product is raw milk for pasteurization, the product may be concentrated by the use of RO or UF membrane filtration without pasteurization, prior to the entrance into the equipment, provided the following sampling, testing, design, installation and operational criteria are met: (1) Prior to processing, all raw milk supplies are sampled and tested for antibiotic residues in accordance with the provisions of Appendix N.; (2) The RO or UF filtration system is designed and operated to assure that milk or milk product temperature is maintained at or below 18.3ºC (65ºF) throughout the process. Provided that the product temperature may rise above 18.3ºC (65ºF) for a period of not more that than fifteen (15) minutes, further provided that should the product temperature rise above 21.1ºC (70ºF), the product shall be either immediately diverted to the system’s balance tank until the product is again below 18.3ºC (65ºF) or diverted to exit the system entirely. Diverted product that has exited the system shall be either discarded, immediately cooled to below 7°C (45°F), or immediately pasteurized; (3) The RO or UF system must be equipped with temperature monitoring and recording devices that comply with the applicable specifications outlined in Appendix H. of this Ordinance. At a minimum, milk or milk product temperature shall be monitored and recorded prior to entering the system, prior to entering each stage of the modules in series that contains cooling, and the retentate stream prior to any final cooler and upon exiting the system; and (4) If the RO or UF system is not designed, installed and operated in accordance with the above noted criteria, the raw milk or milk product must be pasteurized prior to entering the RO or UF system. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 125 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Tech. |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 91, 95, 105, 359 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | PMO, SECTION 7, ITEM 16p.(B), PAGE 91 f. Flow-Promoting Devices: (2) The speed of pumps or other flow-promoting devices, governing the rate of flow through the holding tube, shall be so controlled as to insure the holding of every particle of milk or milk product for at least the time required as defined in Definition FF of this Oridinance…. The metering or timing pump shall be of the positive-displacement type or shall comply with the specifications for magnetic flow meter based timing systems as outlined in Appendix H. The pumps or other flow promoting devices governing the rate of flow through the holding tube shall be a positive displacement type timing pump or shall comply with the specification for magnetic flow meter based timing systems as outlined in Appendix H. Timing pumps and homogenizers, when used as a timing pump, shall not have by-pass lines connected from their outlet pipelines to their inlet pipelines during processing if an additional flow-promoting or vacuum producing device is located within the system. When a homogenizer is used in conjunction with a timing pump it shall be either: PMO, SECTION 7, ITEM 16p.(c), PAGE 95 c. Timing Pump: (1) A positive displacement type timing pump shall be located upstream from the holding tube, or a magnetic flow meter based timing system, which complies with the specifications as outlined in Appendix H, shall be operated to maintain the required rate of milk or milk product flow. The motor of the timing pump shall be connected to the timing pump by means of a common drive shaft, or by means of gears, pulleys or a variable-speed drive, with the gear box, the pulley box or the setting of the variable speed protected in such a manner that the hold time cannot be shortened without detection by the Regulatory Agency.......Variable speed drives, used in connection with the timing pump, shall be so constructed that wearing or stretching of the belt results in a slowdown, rather than a speedup, of the pump. The metering or timing pump shall be of the positive-displacement type or shall comply with the specifications for magnetic flow meter based timing systems. The metering or timing pump shall be of the positive-displacement type or shall comply with the specifications for magnetic flow meter based timing systems. PMO, SECTION 7, ITEM 16p.(e), table 4, PAGE 105 9.3.2 HTST booster pump/metering timing pump PMO, SECTION 7, index, PAGE 359 Metering pump pasteurizer vitamin |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 126 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Tech. |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  | X |  |  |  |  |
|  | FINAL ACTION |  |  |  |  |  | X |  |  |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 92 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | f. Flow-Promoting Devices: (2)The speed of pumps or other flow-promoting devices, …. ….. stretching of the belt results in a slowdown, rather than a speedup of the pump. The metering or timing pump shall be of the positive-displacement type or shall comply with the specifications for magnetic flow meter based timing systems as outlined in Appendix H. Timing pumps and homogenizers, when used as a timing pump, shall not have by-pass lines connected from their outlet pipelines to their inlet pipelines during processing if an additional flow-promoting or vacuum producing device is located within the system. When a homogenizer is used in conjunction with a timing pump, and both are located upstream of the holding tube, it shall be either: ….. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 128 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Other Species |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | PMO-116-117 MMSR-67 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  | X | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | Make the following changes to the 2007 PMO: Strike out text to be deleted and underline text to be added. SECTION 8. ANIMAL HEALTH Page 116 1. All milk for pasteurization shall be from herds in Areas which have a Modified Accredited Advanced Tuberculosis (TB) status or greater higher as determined by the USDA. Provided, that in an Area which fails to maintain such status, any herd shall have been accredited by said Department as tuberculosis free, or shall have passed an annual tuberculosis test, or the Area shall have established a tuberculosis testing protocol for livestock that assures tuberculosis protection and surveillance of the dairy industry within the Area and that it is approved by FDA, USDA and the Regulatory Agency. NOTE: Under the Federal USDA TB Eradication Program, cattle and other hooved mammals (goats, sheep, water buffalo, etc.) are covered within the USDA State TB status determination. 2. All milk for pasteurization shall be from herds under a brucellosis eradication program, which meets one (1) of the following conditions: a. Located in a Certified Brucellosis-Free Area as defined by USDA and enrolled in the testing program for such areas; or b. Meet USDA requirements for an individually certified herd; or c. Participating in a milk ring testing program at least two (2) times per year at approximately one hundred eighty (180) day intervals and all herds with positive milk ring results shall have the entire herd blood tested within thirty (30) days from the date of the laboratory ring tests; or d. Have an individual blood agglutination test annually with an allowable maximum grace period not exceeding two (2) months. NOTE: Under the Federal USDA Brucellosis Eradication Program, only cattle and bison are covered under the USDA State brucellosis status determination. Therefore, other hooved mammals (goats, sheep, water buffalo, etc.) are not covered within the program and must comply with one of the options cited under 3 below. Page 117 3. Goat, sheep, water buffalo, or any other hooved mammal milk for pasteurization, ultra-pasteurization or aseptic processing, defined under this Ordinance, shall be from a herd or flock that: a. Has passed an annual whole herd or flock brucellosis test as recommended by the State Veterinarian or USDA Area Veterinarian in Charge (AVIC); or b. Has passed an initial whole herd brucellosis test, followed only by testing replacement animals or any animals entering the milking group or sold as dairy animals; or c. Has passed an annual random blood-testing program sufficient to provide a confidence level of 99% with a P value of 0.05. Any herd or flock with one (1) or more confirmed positive animals shall go to 100% testing until the whole herd tests show no positive animals are found; or d. Has passed a USDA approved bulk milk test, at USDA recommended frequency, with an implementation date based on availability of the test; or e. Is determined to be free of brucellosis as provided by the development and implementation of a State administered brucellosis-free herd certification program involving a documented surveillance program, which includes records supporting the tests required in this Section, and an official annual written certification from the State Veterinarian documenting their brucellosis-free status. The surveillance program shall be documented and the official annual written State brucellosis-free certification shall be retained on file with the State Regulatory Agency. This official annual written State brucellosis-free certification shall include a current list of Grade “A” non-cattle dairy herds and/or flocks (goats, sheep, water buffalo, etc.) that are covered within the documented surveillance program and contained within the official annual written State brucellosis-free certification. (Refer to the NOTE: on page 26.) …….. Make the following changes to the 2007 MMSR: Strike out text to be deleted and underline text to be added. APPENDIX A. # GUIDELINES FOR COMPUTING ENFORCEMENT RATINGS (FORM FDA 2359j-MILK SANITATION RATING REPORT-SECTION B. REPORT OF ENFORCEMENT METHODS (PAGE 2)) PART I. DAIRY FARMS Page 67 5. Tuberculosis and Brucellosis Certification on file as required (Grade “A” PMO, Section 8 - ANIMAL HEALTH and APPENDIX A. - ANIMAL DISEASE CONTROL). All or nothing Item based on record verification. - Located in a Certified Brucellosis - Free Area as defined by USDA and enrolled in the testing program for such areas; or 1.) Meet USDA requirements for an individually certified herd; or 2.) Participate in an approved milk ring testing program; or 3.) Have individual blood agglutination testing done annually. ; or 4.) For goat, sheep, water buffalo, or any other hooved mammal herds/flocks, excluding cattle and bison, they are included in an official annual written certification from the State Veterinarian documenting their brucellosis-free status. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 129 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | SSCC/Lab |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 132 and 302 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2007 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | Make the following changes to the 2007 PMO: Strike out text to be deleted and underline text to be added. APPENDIX B. MILK SAMPLING, HAULING AND TRANPSORTATION - MILK SAMPLING AND HAULING PROCEDURES EVALUATION OF BULK MILK HAULER/SAMPLER PROCEDURES: Page 132 2. Equipment Requirements: a. Sample rack and compartment to hold all samples collected. b. Refrigerant to hold temperature of milk samples between 0ºC- 4.4ºC (32ºF- 40°F). c. Sample dipper or other sampling devices of sanitary design and material approved by the Regulatory Agency; clean and in good repair. d. Sterile single use sample containers bags, tubes or bottles; properly stored. e. Calibrated pocket thermometer; certified for accuracy every six (6) months; accuracy ± 1°C (2°F). f. Approved sanitizing agent and sample dipper container. g. Watch for timing milk agitation. h. Applicable sanitizer test kit. - APPENDIX J. STANDARDS FOR THE FABRICATION OF SINGLE-SERVICE - CONTAINERS AND CLOSURES FOR MILK AND MILK PRODUCTS ## PURPOSE AND SCOPE Page 302 The use of these Standards will ensure the production of sanitary containers and closures for milk and milk products, as defined in this Ordinance. These Standards shall apply to all blank fabricators, pre-form bottle manufacturers, single-service glass container manufacturers, converters, printers, closure manufacturers, plastic laminators, sheet formers, blow molders, vacuum formers, plastic extruders, injection molders, pre-formers, manufacturers of valves, tubes, dispensing devices, sterile or non-sterile sample containers and any other similar plants. These also apply to fabricating plants producing a component part(s), including fabricators of film and/or closures, which may become a product-contact surface and plants assembling components into a final assembled product. These requirements shall not apply to paper mills or resin manufacturing plants. … |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 131 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Scientific Review |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 170 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | PMO, Appendix D, Pages 170-171 ## IV. CONTINUOUS WATER DISINFECTION A. CHEMICAL DISINFECTION OF WATER Water supplies which are otherwise deemed satisfactory, but which prove unable to meet the bacteriological standards prescribed herein, shall be subjected to continuous disinfection. The individual character of the supply shall be investigated and a treatment program developed, which shall produce a safe supply as determined by bacteriological testing…. Tablet Chlorinator: These hypo-chlorinators inject water into a bed of concentrated calcium hypochlorite tablets. The result is metered into the pump suction line. B. Ultraviolet Light Disinfection of Water The use of ultraviolet (UV) light to disinfect drinking water has been demonstrated to be an effective process is well known as an effective no-disinfection byproduct, environmentally friendly methodology that can inactivate microbes generally targeted by standard chemical disinfectants as well as pathogens that are resistant to other treatments (e.g. such as Cryptosporidium, pseudomonas and others). However, in the design of a water treatment system with UV light, the dairy farm, milk plant, receiving station or transfer station permit holder must exercise care must be exercised to insure that all other requirements of this Ordinance Drinking Water Standards relating to Ssource, Pprotection from Ccontamination and, Cchemical and Pphysical Ccharacteristics are met. Ultraviolet disinfection does will not change the chemical or physical characteristics of the water .( e.g. , such as treat reducing ot or remove removing for high turbidity, mineral levels, or remove arsenic, etc.), so additional treatment, if otherwise dictated, may still be required. Nor does Ultraviolet UV treatment provide residual disinfection. bactericidal action. Some supplies may require routine chemical disinfection, including the maintenance of a residual disinfectant bactericidal agent throughout the distribution system, and there may continue to be a need for the periodic flushing and disinfection of the water distribution system. In addition, the organic nature of materials present in waters can give rise to significant transmission difficulties so that it may be necessary to pretreat some supplies to remove excessive turbidity and color. Color, turbidity, and organic impurities can interfere with the transmission of Ultraviolet UV energy and may decrease the disinfection efficiency below levels required to insure the destruction of pathogenic organisms. In general, color and turbidity measurements do not provide an accurate measure of their impact on UV disinfection efficacy. UV transmissivity (% UVT) multiplied by time accurately measures the impact of water quality on disinfection efficiency. As a result, an on-line in-line Uultraviolet transmissivity (UVT) analyzer may be is needed to assure that the proper dose is provided on a continuing basis; and when water quality changes or it may be necessary to pretreat the water supply to assure consistent water quality. The use of UV to meet the bacteriological requirements of the PMO is acceptable provided the equipment used meets the criteria described herein. Water systems that are within the scope of the U.S. Safe Drinking Water Act as amended and 40 CFR part 141, or State programs that have adopted these requirements shall be regulated under this Act and regulations. Individual water systems that are not regulated under this Act and regulations may be continuously disinfected using UV light based technologies provided the following criteria are met. Criteria for the Acceptability of an Ultraviolet UV Disinfecting Unit - When used to disinfect water to potable drinking water standards, ultraviolet UV light shall be applied so that the entire volume of water receives all particles of water receive at least the following dose: UV at 2,537 Angstrom (254 nanometers) at 186,000 microwatt-seconds per square centimeter or equivalent to achieve an EPA 4 log virus reduction equivalent dose. - A Fflow or time delay mechanism shall be provided so that all water moving past the flow stop or divert valve receives the minimum dose listed required above. - The unit shall be designed to permit the frequent cleaning of the system without disassembly of the unit and shall be cleaned often enough to ensure that the system will provide the required dose at all times. 4. An automatic flow control valve, accurate within the expected pressure range, shall be installed to restrict flow to the maximum design flow of the treatment unit so all particles receive the minimum dose listed above. 5 4. An accurately calibrated ultraviolet UV intensity sensor, properly filtered to restrict its sensitivity to the 2,500-2,800 Angstrom (250-280 nanometers) germicidal spectrum, shall measure the UV energy from the lamps. 6.There shall either be: - Oone (1) sensor for each UV lamp. - The lamps shall be tested every (6) months and the lamp with the lowest output shall be placed in front of the sensor. 7 5. A flow-diversion valve or automatic shut-off valve shall be installed which will permit flow into the potable water lines only when at least the minimum ultraviolet required UV dosage is applied. When power is not being supplied to the unit, the valve shall should be in a closed (fail-safe) position, which will prevents the flow of water into the potable water lines. 6. An automatic flow control valve, accurate within the expected pressure range, shall be installed to restrict flow to the maximum design flow of the treatment unit so that the entire volume of water receives the minimum dose required above. 8 7. The materials of construction shall not impart toxic materials into the water either as a result of the presence of toxic constituents in the materials of construction or as a result of physical or chemical changes resulting from exposure to ultraviolet UV energy. 9 8. The unit shall be designed to protect the operator against electrical shock or UV light exposure. Note: Existing water supplies which otherwise comply with the applicable requirements of this Appendix may continue to use UV disinfection systems that were accepted under M-a-18,. rReplacement systems must comply with this ordinance. until repair or until the first unacceptable water test result. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 132 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Tech. |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | xiv, 217-218, 221-223, 226-230, 232-233, 252-256, 336, 340 and 349 |  |  |  | of the (X - one of the following): |  |  |  |
|  | XX | 2007 PMO |  |  |  | 2007 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | Make the following changes to the 2007 PMO: Strike out text to be deleted and underline text to be added. Page xiv ILLUSTRATIONS Figure 31. Milk-to-Milk Regeneration - Homogenizer Upstream from Holding Tube...…...221 Figure 32. Milk-to-Milk Regeneration - Booster Pump………….........................................221 Figure 33. Milk-to-Milk Regeneration - Homogenizer and Vacuum Chambers Downstream from Flow-Diversion Device ……………………………………………………………..222 Figure 34. HTST System with a Magnetic Flow Meter Using a Constant Speed Centrifugal Pump and a Control Valve ………………………………………………………………..222 Figure 35. HTST System with a Magnetic Flow Meter Using an A-C Variable Speed Centrifugal Pump............…………………………………………………………………223 Figure 36. Controls for Steam Injection Pasteurizer………………………………………...223 Figure 31. HTST Pasteurizer with a Positive Displacement Rotary Timing Pump …… Figure 32. HTST Pasteurizer with a Homogenizer Located at the Outlet of the Heater Section and of a Larger Capacity than Timing Pump ………… Figure 33. HTST Pasteurizer with a Booster Pump, Meter Based Timing System and a Homogenizer with a Bypass Line ………… Figure 34. HTST Pasteurizer with a Booster Pump, Timing Pump and a CIP-Type Separator Located Between Two Pasteurized Product Regenerators with a Pre-Heater……… Figure 35. HTST Pasteurizer with a Booster Pump, Homogenizer as a Timing Pump with an AC Variable Frequency Drive, CIP-Type Separator Located Between Two Pasteurized Product Regenerators and an Air Actuated Discharge Valve with an Air Blow ……… Figure 36. HTST Pasteurizer with a Separator Between the Raw Regenerator and the Heater Section with a Meter Based Timing System and a Regenerator Bypass….. Figure 37. HTST Pasteurizer Utilizing Tubular Tube-In-Tube Type Heat Exchangers and Homogenizer as the Timing Pump Figure 38. HTST Pasteurizer, without a Regenerator or Cooler Section, with a Meter Based Timing System Located Upstream from an Evaporator…… Figure 39. HTST Pasteurizer with a Regenerator, Separator, Skim Surge Tank and a Meter Based Timing System Located Upstream from an Evaporator ….. Figure 40. HHST Pasteurizer with a Flow-Diversion Device Located Downstream of the Cooling Section ……. Figure 41. HHST Pasteurizer Utilizing Steam Injection Heating, Vacuum Flash Cooling and a Flow-Diversion Device Located Downstream of the Cooler Section ….. Figure 42. HHST Pasteurizer Utilizing Direct Culinary Steam Infusion and Vacuum Flash Cooling with a Homogenizer Located Downstream …… Figure 43. HHST Pasteurizer with a Homogenizer as the Timing Pump and Utilizing a Spiral Tubular Heat Exchanger with Indirect Regeneration ….. Figure 44 37. Individual Compression-Type Air Supply……………………………………227 Figure 45 38. Central Compression-Type Air Supply ………………………………………228 Figure 46 39. Individual Blower-Type Air Supply ………………………………………..……..229 Figure 47 40. Individual Fan-Type Air Supply...…………………………………...……….229 Figure 48 41. Rotating Mandrel Assembly ….......………....…………………………….....230 Figure 49 42. Culinary Steam Piping Assembly for Steam Infusion or Injection……………232 Figure 50 43. Culinary Steam Piping Assembly for Steam Infusion or Injection (Optional Configuration)...….………………………………………………...…………..233 Figure 51 44. Culinary Steam Piping Assembly for Airspace Heating or Defoaming…........233 Figure 52 45. Logic Diagram: HTST Flow-Diversion Device, Divert Valve Stem……..…..252 Figure 53 46. Logic Diagram: HTST Flow-Diversion Device, Leak Detect Valve Stem...... 253 Figure 54 47. Logic Diagram: HTST Safety Limit Recorder-Controller...…......................…254 Figure 55 48. Logic Diagram: HTST Timing Pump...….................................………………255 Figure 56 49. Logic Diagram: HTST Booster Pump...…................................…………........256 Figure 57 50. Pressure Switch Setting…...……….....................…………………….....……299 Figure 58 51. Vitamin Fortification…..……….…………................…………………….….340 Figure 59 52. Decision Tree for Using pH, aw, or the Interaction of pH and aw to Determine if a Milk or Milk Product Requires Time/Temperature for Safety……………….……..….349 NOTE: Re-number the ILLUSTRATIONS’ Section page references accordingly. aPPENDIX H. PASTEURIZATION EQUIPMENT AND PROCEDURES AND OTHER EQUIPMENT I. HTST pasteurization Page 217 PRESSURE RELIEF VALVE, LOCATED WITHin HTST, HHST AND ASEPTIC PROCESSING SYSTEMS 1. Between the Timing Pump and the Beginning of the Holding Tube: Placement of a pressure relief valve between the timing pump and the beginning of the holding tube is acceptable provided it meets either OPTION I or II below: Option I: a. Provisions are made for the cleaning of the valve vent and any return piping to the constant-level tank whenever the system is cleaned. b. The pasteurizer shall not be timed if the valve is leaking. Leakage may be determined by observation at the pressure relief valve vent opening to the floor or at the opening of the return piping from the pressure relief valve vent into the constant-level tank. c. The system is designed and operated so that loss of pressure from the pasteurized side of the regenerator cannot occur if the system flow-promoting devices stop while the FDD is in the forward-flow position. A system not protected against this potential pressure loss is considered a violation of Item 16p(D) of this Ordinance. For Example: In a magnetic flow meter based timing system there is a fail-safe, spring-to-close valve or check-valve that must also be located between the timing pump and the holding tube. Item 16p(D) of this Ordinance is satisfied if the pressure relief valve is located prior to this fail-safe valve or check-valve. Option 2. The pressure relief valve is spring-loaded and plumbed so that it cannot be opened or forced open in any mode, “Product”, “CIP” or “Inspect”, without the assistance of pressure from the liquid flowing through the system. In this case, a leaking pressure relief valve can cause an unacceptable loss of pressure in the pasteurized side of the regenerator if the system flow-promoting devices stop while the FDD is in the forward-flow position. This is considered a violation of Item 16p(D) of this Ordinance. Any leakage from this pressure relief valve must be readily visible. This may be accomplished by opening the pressure relief valve vent directly to the floor or by providing sanitary piping from the pressure relief valve vent to the constant-level tank. If the later option is utilized, the piping shall be properly sloped to assure drainage to the constant-level tank and shall be provided with a properly located and installed sight-glass. Page 218 2. Downstream from the Holding Tube in HTST Systems: Page 221 Figure 31. Milk-to-Milk Regeneration – Homogenizer Upstream from Holding Tube HTST Pasteurizer with a Positive Displacement Rotary Timing Pump Figure 32. Milk-to-Milk Regeneration - Booster Pump HTST Pasteurizer with a Homogenizer Located at the Outlet of the Heater Section and of a Larger Capacity than the Timing Pump Page 222 NOTE: Delete EXISTING DRAWING AND replace with THE new drawing. Figure 33. Milk-to-Milk Regeneration – Homogenizer and Vacuum Chambers Downstream from Flow-Diversion Device HTST Pasteurizer with a Booster Pump, Meter Based Timing System and a Homogenizer with a Bypass Line NOTE: Delete EXISTING DRAWING AND replace with THE new drawinG. Figure 34. HTST System with a Magnetic Flow Meter Using a Constant Speed Centrifugal Pump and a Control Valve HTST Pasteurizer with a Booster Pump, Timing Pump and a CIP-Type Separator Located Between Two Pasteurized Product Regenerators with a Pre-Heater Page 223 NOTE: Delete EXISTING DRAWING AND replace with THE new drawing. Figure 35. HTST System with a Magnetic Flow Meter Using an A-C Variable Speed Centrifugal Pump HTST Pasteurizer with a Booster Pump, Homogenizer as a Timing Pump with an AC Variable Frequency Drive, CIP-Type Separator Located Between Two Pasteurized Product Regenerators and an Air Actuated Discharge Valve with an Air Blow NOTE: Delete EXISTING DRAWING AND replace with THE new drawing. Figure 36. Controls for Steam Injection Pasteurizer HTST Pasteurizer with a Separator Between the Raw Regenerator and the Heater Section with a Meter Based Timing System and a Regenerator Bypass NOTE: ADD THE following new drawingS on the next pages of the pmo. Figure 37. HTST Pasteurizer Utilizing Tubular Tube-In-Tube Type Heat Exchangers and a Homogenizer as the Timing Pump Figure 38. HTST Pasteurizer, without a Regenerator or Cooler Section, with a Meter Based Timing System Located Upstream from an Evaporator Figure 39. HTST Pasteurizer with a Regenerator, Separator, Skim Surge Tank and a Meter Based Timing System Located Upstream from an Evaporator Figure 40. HHST Pasteurizer with a Flow-Diversion Device Located Downstream of the Cooling Section Figure 41. HHST Pasteurizer Utilizing Steam Injection Heating, Vacuum Flash Cooling and a Flow-Diversion Device Located Downstream of the Cooler Section Figure 42. HHST System Utilizing Direct Culinary Steam Infusion and Vacuum Flash Cooling with a Homogenizer Located Downstream Figure 43. HHST Pasteurizer with a Homogenizer as the Timing Pump and Utilizing a Spiral Tubular Heat Exchanger with Indirect Regeneration II. AIR FOR DRYING EQUIPMENT AND AIR UNDER PRESSURE – DIRECT CONTACT WITH MILK AND MILKPRODUCTS AND MILK PRODUCT-CONTACT SURFACES FABRICATION AND INSTALLATION Page 226 Air Piping:…… a. When air under pressure is directed at product-contact surfaces of containers, closures and supplementary fitments, the air passage from the final filter to the point of application shall be made of a non-toxic, relatively nonabsorbent material. In this application, check-valves are not required. The final filter shall be located as close as practical to the point of application. (Refer to Figure 41 48) Page 227 Figure 44 37. Individual Compression-Type Air Supply Page 228 Figure 45 38. Central Compression-Type Air Supply Page 229 Figure 46 39. Individual Blower-Type Air Supply Figure 47 40. Individual Fan-Type Air Supply Page 230 Figure 48 41. Rotating Mandrel Assembly Page 232 Figure 49 42. Culinary Steam Piping Assembly for Steam Infusion or Injection Page 233 Figure 50 43. Culinary Steam Piping Assembly for Steam Infusion or Injection Optional Configuration) Figure 51 44. Culinary Steam Piping Assembly for Airspace Heating or Defoaming VI. CRITERIA FOR THE EVALUATION OF COMPUTERIZED SYSTEMS FOR GRADE “A” PUBLIC HEALTH CONTROLS Page 252 Figure 52 45. Logic Diagram: HTST Flow-Diversion Device, Divert Valve Stem Page 253 Figure 53 46. Logic Diagram: HTST Flow-Diversion Device, Leak-Detect Valve Stem Page 254 Figure 54 47. Logic Diagram: HTST Safety Limit Recorder-Controller Page 255 Figure 55 48. Logic Diagram: HTST Timing Pump Page 256 Figure 56 49. Logic Diagram: HTST Booster Pump APPENDIX I. PASTEURIZATION EQUIPMENT AND CONTROLS – TESTS II. TEST PROCEDURES TEST 13. SETTING OF CONTROL SWITCHES FOR MILK OR MILK PRODUCT PRESSURE IN THE HOLDING TUBE Page 298 Procedure: 1. From Figure 50 57 determine the pressure switch setting necessary for the operating temperature, not the diversion temperature, being used in the process. Install the sanitary pressure gauge, of known accuracy, and the pressure switch sensing-element on the pneumatic testing device…. 4. Determine that the cut-in pressure on the switch is equivalent to or greater than the required pressure from Figure 50 57. If adjustment is necessary, refer to the manufacturer's instructions. Page 299 Figure 57 50. Pressure Switch Setting APPENDIX O. VITAMIN FORTIFICATION OF FLUID MILK PRODUCTS PROCESS/METHODS OF VITAMIN ADDITION Page 336 1. The first is a piston type metering pump with valves…. The recommended injection point is after separation and prior to homogenization. This allows the homogenization process to distribute the vitamins throughout the milk. A check-valve is recommended to prevent milk from contaminating the vitamin concentrate. Separate pumps, tubing and check-valves are recommended when multiple types of vitamin concentrates are injected. (Refer to Figure 51 58) Page 340 NOTE: Figure 51 58 details a two (2) speed vitamin fortification installation using two (2) pumps and two (2) vitamin concentrate sources. This enables changing from different vitamin concentrates and different speed pumps via the adjustment of three-way valves. Figure 58 51. Vitamin Fortification APPENDIX R. DETERMINATION OF TIME/TEMPERATURE CONTROL FOR SAFETY milk AND MILK PRODUCTS iNSTRUCTION FOR USING TABLES A AND B Page 349 Figure 59 52. Decision Tree for Using pH, aw, or the Interaction of pH and aw to Determine if a Milk or Milk Product Requires Time/Temperature for Safety NOTE: The following text is not to be inserted into the PMO or any other NCIMS document: The new Figure 31. HTST Pasteurizer with a Positive Displacement Rotary Timing Pump and Figure 32. HTST Pasteurizer with a Homogenizer at the Outlet of the Heater Section and of a Larger Capacity than the Timing Pump are attached to this Proposal as an example of the drawings that we are proposing to be added to the PMO. The remaining eleven (11) proposed new Figures were not available by the Proposal submission due date. They are currently being finalized and will be provided to the voting delegates prior to the NCIMS Conference. The proposed new Figures will use the flow patterns and equipment configurations shown in illustrations in the current 3-A Sanitary Accepted Practice 603-07-3-A Accepted Practices for the Construction, Installation, Testing and Operation of High-Temperature-Short-Time and Higher-Heat-Shorter-Time Pasteurizer Systems. Section B of this Proposal provides a listing of the specific thirteen (13) 3-A Illustrations from which the new proposed PMO Figures will be drawn. These Figures will be put into PMO format as shown in Figures 31 and 32 and those parts of the schematic drawings that illustrate a 3-A requirement that are not required in the PMO, such as pressure differential controls on the cooling section, will be deleted from the 3-A drawings and will not be included in the new PMO Figures. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 134 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Tech. |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 220 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | After the last paragraph on page 220, add the following: The Use of Vacuum Breakers on HTST Systems Vacuum breakers are often used on HTST systems to help maintain proper pressure relationships in milk-to-milk regenerator sections, or to prevent a negative pressure between the FDD and any downstream flow-promoting device vacuum equipment. The use of vacuum breakers on HTST systems is allowed provided the following conditions are met: 1) Vacuum breakers must open to the atmosphere when subject to a negative pressure 2) Spring-to-close loaded vacuum breakers are not allowed 3) 2) The pasteurized milk and milk product, between its outlet from the regenerator and the nearest point downstream open to the atmosphere, shall rise to a vertical elevation of 30.5 centimeters (12 inches) above the highest raw milk or milk product level, downstream from the constant-level tank, and shall be open to the atmosphere at this or a higher elevation. Spring-to-close vacuum breakers are not allowed. |  |  |  |  |  |  |  |  |  |  |

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Proposal #: |  |  |  | 137 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Committee: |  |  |  | Tech. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | No Action |  |  |  |  |  |  | Passed as Submitted |  |  |  |  | Passed as Amended |  |  |  |  |  |  |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Changes to be made on page(s): |  |  |  |  |  |  |  |  | 259 |  |  |  |  |  |  |  |  |  | of the (X - one of the following): |  |  |  |  |  |  |  |  |  |  |  |
| X |  | 2007 PMO |  |  |  |  |  |  |  |  |  | 2007 EML |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  |  |  |  |  |  |  | 2400 Forms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  |  |  |  |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Changes to be made to the 2007 PMO: Strike out text to be deleted and underline to be added. PAGE 259 APPENDIX H. PASTEURIZATION EQUIPMENT AND PROCEDURES VIII MILK AND MILK PRODUCT CONTINUOUS-FLOW (HTST AND HHST) PASTEURIZATION---CCP Model HACCP Plan Summary Under Monitoring and the "What" column, make the following changes: Residence time in the holding tube in continuous-flow pasteurizers with magnetic flow meter based timing systems. Flow rate in forward flow in the holding tube (to verify minimum holding time) in continuous flow pasteurizers with magnetic flow meter based timing systems. (Note see attached CCP Model to view this change) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX H. PASTEURIZATION EQUIPMENT AND PROCEDURES

VIII MIlk and Milk Product CONTINUOUS-FLOW (HTST and HHST) Pasteurization---

CCP Model HACCP Plan Summary

| Critical Control Point (CCP) | Hazard(s) | ## Critical Limits | # Monitoring |  |  |  | Corrective Action(s)* | ###### CCP ###### Verification** and *** | Records |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | What | How | Frequency | Who |  |  |  |
| Milk and Milk Products Pasteurization (HTST and HHST) | Biological- Vegetative Pathogens (non-spore formers) | Time and Temperature Note: Assuring that the minimum holding times are met in systems which use a sealed timing pump would be as CCP verification during required equipment calibration. | Temperature at the exit of the holding tube Residence time in the holding tube in continuous-flow Pasteur-izers with magnetic flow meter based timing systems Flow rate in forward flow in the holding tube (to verify minimum holding time) in continuous flow pasteurizers with magnetic flow meter based timing systems, . | Temp. Recorder Chart Flow Recorder Chart | Continuous during Operation Continuous during Operation | Pasteurizer Operator Pasteurizer Operator | Manually divert flow of product Isolate the affected product Evaluate and determine disposition of the product (reprocess or disposal) Document actions | Record Review: Pasteurizer charts verified Equipment Function Checks: Operator performs required daily tests and record on the temperature charts. Authorized plant person (supervised by regulatory when required) conducts checks listed in the Milk Plant Equipment Test Report (FDA Form 2359b). Seals:Verify required regulatory seals daily | Pasteurizer Charts Corrective Action Records CCP Verification- Records, including equipment testing records |

* A properly operating HTST or HHST pasteurization system will divert raw product to the constant-level tank when predetermined set points are not met.

**Every particle of milk or milk is heated, in a properly designed, calibrated and operated pasteurizer, to one of the temperature and time combinations specified in the current Grade "A" PMO.

*** Pressures in the regenerator of continuous-flow pasteurizers, and in the case of HHST pasteurizers as required in the holding tubes, across steam injectors,

and within infusion chambers shall be addressed in the HACCP Plan and managed as CCP verification(s).

Product Description: ___________________________________ Method of Storage and Distribution: ______________________

Intended Use and Consumer: ____________________________

Signature: ________________________________________           Date: __________________

|  | 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS |  |  |  |  |  |  | Proposal #: |  | 139 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Committee: |  | Tech. |  |
|  |  |  |  |  | No Action |  | Passed as Submitted |  | Passed as Amended |  |  |
|  | COUNCIL ACTION |  |  |  |  |  |  |  | X |  |  |
|  | FINAL ACTION |  |  |  |  |  |  |  | X |  |  |
| C. Proposed Solution |  |  |  |  |  |  |  |  |  |  |  |
|  | Changes to be made on page(s): |  |  | 264 & 265 |  |  |  | of the (X - one of the following): |  |  |  |
|  | X | 2007 PMO |  |  |  | 2005 EML |  |  |  |  |  |
|  |  | 2007 MMSR |  |  |  | 2400 Forms |  |  |  |  |  |
|  |  | 2007 Procedures |  |  |  | 2007 Constitution and Bylaws |  |  |  |  |  |
|  | PMO APPENDIX H, IV. THERMOMETER SPECIFICATIONS, PAGE 234-242 ## INDICATING THERMOMETERS FOR BATCH PASTEURIZERS Type: 2. Digital: Stand Alone: c. The electromagnetic compatibility of this device for this use shall be documented and available to the Regulatory Agency. The device must be tested to determine the effects of electrostatic discharge, power fluctuation, conductive emission and susceptibility, and radiative emission and susceptibility. The device must comply with the requirements for performance level characteristics of industrial devices. Vendors shall develop protocols for these tests with FDA concurrence. d. The effect of exposure to specific environmental conditions shall be documented. The device must be tested to determine the effects of low and high temperatures, thermal shock, humidity, physical shock and salt fog. Vendors shall develop protocols for these tests with FDA concurrence. 3. Digital: Combination: c. The electromagnetic compatibility of this device for this use shall be documented and available to the Regulatory Agency. The device must be tested to determine the effects of electrostatic discharge, power fluctuation, conductive emission and susceptibility, and radiative emission and susceptibility. The device must comply with the requirements for performance level characteristics of industrial devices. Vendors shall develop protocols for these tests with FDA concurrence. d. The effect of exposure to specific environmental conditions shall be documented. The device must be tested to determine the effects of low and high temperatures, thermal shock, humidity, physical shock and salt fog. Vendors shall develop protocols for these tests with FDA concurrence. INDICATING THERMOMETERS LOCATED ON PASTEURIZATION PIPELINES Type: 2. Digital: c. The electromagnetic compatibility of this device for this use shall be documented and available to the Regulatory Agency. The device must be tested to determine the effects of electrostatic discharge, power fluctuation, conductive emission and susceptibility, and radiative emission and susceptibility. The device must comply with the requirements for performance level characteristics of industrial devices. Vendors shall develop protocols for these tests with FDA concurrence. d. The effect of exposure to specific environmental conditions shall be documented. The device must be tested to determine the effects of low and high temperatures, thermal shock, humidity, physical shock and salt fog. Vendors shall develop protocols for these tests with FDA concurrence. AIRSPACE INDICATING THERMOMETER FOR BATCH PASTEURIZERS Type: 2. Digital: Stand Alone: - The electromagnetic compatibility of this device for this use shall be documented and available to the Regulatory Agency. The device must be tested to determine the effects of electrostatic discharge, power fluctuation, conductive emission and susceptibility, and radiative emission and susceptibility. The device must comply with the requirements for performance level characteristics of industrial devices. Vendors shall develop protocols for these tests with FDA concurrence. - The effect of exposure to specific environmental conditions shall be documented. The device must be tested to determine the effects of low and high temperatures, thermal shock, humidity, physical shock and salt fog. Vendors shall develop protocols for these tests with FDA concurrence. 3. Digital: Combination: c. The electromagnetic compatibility of this device for this use shall be documented and available to the Regulatory Agency. The device must be tested to determine the effects of electrostatic discharge, power fluctuation, conductive emission and susceptibility, and radiative emission and susceptibility. The device must comply with the requirements for performance level characteristics of industrial devices. Vendors shall develop protocols for these tests with FDA concurrence. d. The effect of exposure to specific environmental conditions shall be documented. The device must be tested to determine the effects of low and high temperatures, thermal shock, humidity, physical shock and salt fog. Vendors shall develop protocols for these tests with FDA concurrence. TEMPERATURE-RECORDING DEVICES FOR BATCH PASTEURIZERS 1. UTILIZING TEMPERATURES LESS THAN 71oC (160oF) Chart Scale: …. Pen-Arm Setting Device: Easily accessible and simple to adjust for mercury-actuated recording thermometer. (Refer to Appendix I., Test 4) Temperature Sensing Device: Protected against damage at a temperature of 105°C (220°F). Mercury-Actuated: Bulb, tube, and spring, protected against damage at a temperature of 105°C (220°F). Digital: a. No more than 0.5ºC (1.0 ºF) drift over three (3) months use on a batch pasteurizer compared to a certified temperature source. b. Self-diagnostic circuitry, which provides constant monitoring of all sensing, input and conditioning circuits. The diagnostic circuitry should be capable of detecting “open” circuits, “short” circuits, poor connections and faulty components. Upon detection of failure of any component, the device shall blank, become unreadable or go visibly out of range. c. The electromagnetic compatibility of this device for this use shall be documented and available to the Regulatory Agency. The device must be tested to determine the effects of electrostatic discharge, power fluctuation, conductive emission and susceptibility, and radiative emission and susceptibility. The device must comply with the requirements for performance level characteristics of industrial devices. d. The effect of exposure to specific environmental conditions shall be documented. The device must be tested to determine the effects of low and high temperatures, thermal shock, humidity, physical shock and salt fog. e. Both the probe and the display case shall be constructed so that they may be sealed by the Regulatory Agency. f. Calibration of the device shall be protected against unauthorized changes. g. The device shall be protected against unauthorized component or sensing element replacement. Replacement of any component or sensing element shall be regarded as a replacement of the indicating thermometer and subject to Regulatory Agency inspection and all application tests under Appendix I. of this Ordinance. h. The sensing element shall be encased in appropriate material constructed in such a way that the final assembly meets the conditions of Item 11p of this Ordinance. Chart Speed: A circular chart shall make one (1) revolution in not more than twelve (12) hours. Two (2) charts shall be used if operations extend beyond twelve (12) hours in one (1) day. Circular charts shall be graduated for a maximum record of twelve (12) hours. Strip-charts may show a continuous recording over a twenty-four (24) hour period. ## 2. UTILIZING TEMPERATURES GREATER THAN 71ºC (160ºF) Batch pasteurizers used solely for thirty (30) minute pasteurization of milk and milk products at temperature above 71°C (160°F) may use temperature-recoding devices that comply with 1. with the following options: Chart Scale: …. Digital Temperature Sensing Device: a. No more than 1°C (2°F) drift over three (3) months use on a batch pasteurizer compared to a certified temperature source. Chart Speed: …. RECORDER/CONTROLLERS FOR CONTINUOUS PASTEURIZERS Pen-Arm Setting Device: Easily accessible and simple to adjust for mercury-actuated recording thermometer. (Refer to Appendix I., Test 4) Temperature Sensing Device: Mercury-Actuated: Bulb, tube, or and spring, or thermistor, protected against damage at a temperature of 105°C (220°F). Provided, that the recorder/controller temperature sensing devices, used on HHST systems, shall be protected against damage at temperatures of 149°C (300°F). Digital: a. No more than 0.5°C (1.0°F) drift over three (3) months use on a HTST system compared to a certified temperature source. b. Self-diagnostic circuitry, which provides constant monitoring of all sensing, input and conditioning circuits. The diagnostic circuitry should be capable of detecting “open” circuits, “short” circuits, poor connections and faulty components. Upon detection of failure of any component, the device shall blank or become unreadable. c. The electromagnetic compatibility of this device for this use shall be documented and available to the Regulatory Agency. The device must be tested to determine the effects of electrostatic discharge, power fluctuation, conductive emission and susceptibility, and radiative emission and susceptibility. The device must comply with the requirements for performance level characteristics of industrial devices. d. The effect of exposure to specific environmental conditions shall be documented. The device must be tested to determine the effects of low and high temperatures, thermal shock, humidity, physical shock and salt fog. e. Both the probe and the display case shall be constructed so that they may be sealed by the Regulatory Agency. f. Calibration of the device shall be protected against unauthorized changes. g. The device shall be protected against unauthorized component or sensing element replacement. Replacement of any component or sensing element shall be regarded as a replacement of the indicating thermometer and subject to Regulatory Agency inspection and all applicable tests under Appendix I. of this Ordinance. h. The sensing element shall be encased in appropriate material constructed in such a way that the final assembly meets the conditions of Item 11p of this Ordinance. i. The device must be tested from the sensing probe through the final output. Stem Fitting: A Ppressure-tight seat against the inside wall of the pipe; no threads exposed to milk or milk products; and the distance from the underside of the ferrule to the sensitive portion of the bulb is to be not less than 76 millimeters (3 inches). TEMPERATURE-RECORDING DEVICES USED IN STORAGE TANKS Chart Scale: …. TEMPERATURE-RECORDING DEVICES ON CLEANING SYSTEMS Chart Scale: …. PMO, APPENDIX H, PAGE 263 - TEST 2. - RECORDING THERMOMETERS - TEMPERATURE ACCURACY Application: To all mercury-actuated recording and recorder-controller thermometers controllers used to record milk or milk product temperatures during pasteurization or aseptic processing. NOTE: When this Test is performed on mercury-actuated recorder-controllers used with HHST pasteurization or aseptic processing systems that operate at or above the boiling point of water, an oil or other suitable media bath shall be substituted for the processing (operating) temperature water mentioned in Procedures 1, 4, 5, 6, and 7 as well as the boiling water mentioned in Procedures 2, 3 and 5. The temperature of the oil bath that is used in place of the boiling water shall be above the normal operating range but below the highest temperature division on the chart. Method: The testing of a mercury-actuated recording thermometer for temperature accuracy involves the determination of whether or not the temperature pen-arm will return to within 0.5ºC (1ºF), or 1°C (2°F) as provided in the Criteria above, of its previous setting, after exposure to high heat and melting ice. Procedure: 1. For all recording thermometers -Adjust the recording pen to read exactly as the previously tested indicating thermometer, in the temperature range for the process being used, after a stabilization period of five (5) minutes, two (2) minutes for electronic recording thermometers, at a constant temperature. The bath shall be rapidly agitated throughout the stabilization period. 2. For capillary thermometers only -Prepare a second media bath by heating to the boiling point, or in the case of HHST or aseptic systems, to a temperature above the normal operating range but below the highest temperature division on the chart, and maintain temperature. Prepare a third container with melting ice. Place all media baths within working distance of the temperature-sensing element(s). 3. Immerse the recording thermometer sensing element into the boiling water, or in the case of HHST or aseptic processing systems into the media bath described above, for not less than five (5) minutes, two (2) minutes for electronic recording thermometers. 4. Remove the recording thermometer sensing element from the boiling water or other media bath and immerse it in the media bath at a temperature within the temperature range for the process being used. Allow a five (5) minute, two (2) minutes for electronic recording thermometers, stabilization period for both indicating and recording thermometers. Compare readings of the indicating and recording thermometers. The recording thermometer reading should be within ±0.5ºC (±1ºF) or ± 1°C (±2°F) as provided above, of the indicating thermometer reading. 5. Remove the recording thermometer sensing element from the bath in the temperature range for the process being used, and immerse in melting ice for not less than five (5) minutes, two (2) minutes for electronic recording thermometers. 6. Remove the recording thermometer-sensing element from the ice water and immerse in a bath at a temperature, range for the process being used. Allow a five (5) minute, two (2) minutes for electronic recording thermometers, stabilization period for both indicating and recording thermometers. Compare readings of the indicating and recording thermometers. The recording thermometer reading should be within ±0.5ºC (±1ºF), or ± 1°C (±2°F) as provided above, of the indicating thermometer reading. 7. Re-seal the regulatory controls as necessary and record the indicating and recording thermometer readings obtained from Procedures 1, 4, and 6. PMO, APPENDIX H, PAGE 266 - TEST 4. RECORDING THERMOMETERS - CHECK AGAINST INDICATING THERMOMETERS Corrective Action: If the mercury-actuated recording thermometer or recorder-controller thermometer reads higher than the indicating thermometer, the pen or temperature adjusting mechanism shall be adjusted by the operator. If the digital recording thermometer or recorder-controller thermometer reads higher than the indicating thermometer, the recording temperature should be adjusted to agree with the indicating thermometer. Retest the thermometer after adjustment. 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS; Proposal #:; 141. Committee:; Tech.. No Action; Passed as Submitted; Passed as Amended. COUNCIL ACTION; X. FINAL ACTION; X. C. Proposed Solution. Changes to be made on page(s):; 265, 278, and 300; of the (X - one of the following):. X; 2007 PMO; 2005 EML. 2007 MMSR; 2400 Forms. 2007 Procedures; 2007 Constitution and Bylaws. Make the following changes to the 2007 PMO: Strike out text to be deleted and underline text to be added. TEST 3. RECORDING THERMOMETERS - TIME ACCURACY Page 265 Application: To all recording and recorder-controller thermometers used to record the time of pasteurization or aseptic processing, including those used to record flow rates in magnetic flow meter based timing systems. 9.3 ADDITIONAL HTST TESTS FOR BOOSTER PUMPS – INTERWIRING Page 278 Application: To all booster pumps used for HTST systems where the FDD is located immediately after the holding tube. , except for those systems which are magnetic flow meter based timing systems, Test 9.3.2 is not required to be performed. TEST 15. ELECTRO-MAGNETIC INTERFERENCE FROM HAND-HELD COMMUNICATION DEVICES Page 300 Application: To all electronic controls devices used to assure compliance with public health safeguards on continuous flow pasteurization and aseptic processing equipment that are installed in milk plants where hand-held communication devices are used. Frequency: Upon installation; any alteration of the electronic controls devices; every three (3) months thereafter; and whenever the type or wattage of the hand-held communication device(s) used in that facility milk plant is changed. Once a hand-held communication device has been shown to cause a given electronic control device to react adversely, the electronic control device must be repaired and re-tested using the same type hand-held communication device Test does not have to be repeated every three (3) months using that specific hand-held communication device on the adversely affected electronic control device. (Refer to the NOTE: below.) If the any electronic control device is altered or there is a change in the hand-held communication device(s) used, the electronic control device(s) would be required to be tested. Criteria: The use of hand-held communication devices shall not have no any adverse effect on the electronic control device’s public health safeguards. Apparatus: One (1) hand-held communication device representing each make and model used in the facility milk plant. The device must be operating at maximum output, and be fully charged. Method: By observing the actual effect of the hand-held communication device on an electronic control device, it can be determined if that hand-held communication device can be used near that equipment without compromising any of the electronic control device’s public health safeguard. Procedure: 1. Position the hand-held communication device 30.5 centimeters (12 inches) in front of the electronic control device where the public health safeguard(s) resides. 2. Place the hand-held communication device in the “send” mode for five (5) seconds and observe the effect on the electronic control device’s public health safeguard(s). There should not be no any adverse effect with the electronic control device. An adverse effect is any change that may adversely affect an electronic control device’s public health safeguard(s). 3. If applicable, repeat the Test with the operator access door open. 4. Repeat the above Test for each hand-held communication device identified in the Apparatus Section. 5. Repeat the Test for each electronic control device used to regulate a pasteurization or aseptic processing system’s public health safeguard(s). For Example: For the temperature set point, operate the pasteurizer or aseptic processor pasteurization or aseptic processing equipment on water in diverted-flow in the “Product” mode, at a steady temperature within 3ºC (5ºF) of the lowest cut-in temperature. In this example, an adverse effect is defined as the forward-flow movement of the FDD or any artificial increase in temperature. Corrective Action: Have the facility milk plant check for shielding, grounding and other installation concerns with the electronic control device and retest. Until a solution, acceptable to the Regulatory Agency, can be found that does not adversely affect the electronic control device’s public health safeguard(s), the hand-held communication device may not cannot be used in the area of the electronic control device’s public health safeguard(s). NOTE: Continuous “Hand-Held Communication Device Free” or “Radio Free” zones, etc., are not acceptable permanent solutions to hand-held communication devices which cause adverse affects to an electronic control device’s public health safeguards.. 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS; Proposal #:; 142. Committee:; Tech.. No Action; Passed as Submitted; Passed as Amended. COUNCIL ACTION; X. FINAL ACTION; X. C. Proposed Solution. Changes to be made on page(s):; 268, 269, & 270; of the (X - one of the following):. X; 2007 PMO; 2005 EML. 2007 MMSR; 2400 Forms. 2007 Procedures; 2007 Constitution and Bylaws. Criteria: The FDD shall function correctly in operating situations and shall de-energize the timing pump and all other flow-promoting devices capable of causing flow through the FDD, in the event of malfunction or incorrect assembly. 5.1 LEAKAGE PAST VALVE SEAT(S) Apparatus: Suitable tools for the disassembly of the FDD and the sanitary piping. Method: Observe the valve seat(s) of the FDD for leakage. Procedure: 1. With the system operating on water, place the FDD in the diverted-flow position. 2. For single stem FDDs, disconnect the forward-flow piping and observe the valve seat for leakage. Check the leak escape ports to see if they are open. 3. For dual stem FDDs, observe the leak-detect line discharge or sight glass for leakage. Corrective Action: If leakage is noted, the FDD must be dismantled and defective gaskets replaced or other suitable repairs made. 5.2 OPERATION OF VALVE STEM(S) Apparatus: Suitable tools for tightening the packing nut on the stem(s) Method: Observe the FDD valve stem(s) for ease of movement. Procedure: When a stem-packing nut is used, tighten it as much as possible. Operate the system at maximum normal operating pressure and place the FDD in forward and diverted-flow several times. Note the freedom of action of the valve stem. Corrective Action: If the valve action is sluggish, suitable adjustment or repair shall be made. The stem shall move freely in all positions, when the stem-packing nut is fully tightened. 5.3 DEVICE ASSEMBLY - SINGLE STEM DEVICE Apparatus: Sanitary fitting wrench. Method: When the FDD is improperly assembled and in diverted-flow (below cut-out temperature), observe the function of the timing pump and all other flow-promoting devices capable of causing flow through the FDD. Procedure: 1. With the system in operation below the cut-in temperature, unscrew by one-half turn, the 13H hex nut that holds the top of the valve to the valve body. This should de-energize the timing pump and all other flow-promoting devices, which are capable of causing flow through the FDD. This test shall be conducted without piping connected to the forward-flow port of the FDD. (This allows movement of the top of the valve when the hex nut is loosened.) Re-tighten the 13H hex nut. 2. With the system in operation below the cut-in temperature, remove the connecting key, located at the base of the valve stem. The timing pump and all other flow-promoting devices, which are capable of causing flow through the FDD should be de-energized. Also, the isolation valves of any downstream vacuum source must be de-energized and in the closed position. 3. Attempt to restart the timing pump and each flow-promoting device capable of causing flow through the FDD. None of these flow-promoting devices should start or operate. Corrective Action: If any flow-promoting device fails to respond as indicated, immediate checks of the device assembly and wiring are required to locate and correct the cause. 267 5.4 DEVICE ASSEMBLY - DUAL STEM DEVICE NOTE: The test procedure presented in this Section is typical of tests accepted by FDA for various specific types of FDDs. Testing details, which may vary, are provided in individual FDD operator’s manuals that have been reviewed by FDA and are specified by part number in FDA’s Coded Memoranda (M-b’s). In each of these FDA accepted test methods, if the words "metering pump" or "timing pump" are used they shall be understood to mean "timing pump and all other flow-promoting devices, which are capable of causing flow through the FDD". Apparatus: None Method: Observe the function of the timing pump and all other flow-promoting devices, which are capable of causing flow through the FDD when the FDD is improperly assembled. Procedure: 1. With the FDD in diverted-flow, caused by temperature, and the FDD properly assembled, move the FDD to the forward-flow position and disconnect the stem from the actuator. 2. Move the FDD to the diverted-flow position and turn on the timing pump and all other flow- promoting devices, which are capable of causing flow through the FDD. The timing pump and all other flow-promoting devices must be de-energized and must not run. If any pump starts momentarily and then stops, it may indicate the improper wiring of the one (1) second time delay as allowed in 16p(B)2.b.(10). Separators or downstream vacuum sources must be effectively valved-out of the system. Also, the isolation valves of any downstream vacuum source must be de-energized and in the closed position. 3. Reassemble the FDD by moving it to the forward-flow position and reconnecting the stem to the actuator. 4. Repeat the procedure for the other actuator. Corrective Action: If any of the flow-promoting devices fail to respond as indicated, an immediate check of the FDD assembly and wiring is required to locate and correct the cause. 5.5 MANUAL DIVERSION (Booster pump installed in the HTST system) Apparatus: None Method: Observe the response of the system to manual diversion. Procedure: 1. With the HTST system in operation and the FDD in the forward-flow position, press the manual diversion button. This should: a. Cause the FDD to assume the divert position; b. De-energize the booster pump; c. any downstream vacuum sources are effectively valved out Also, the isolation valves of any downstream vacuum source must be de-energized and in the closed position; d. The pressure differential between raw and pasteurized milk or milk product in the regenerator should be maintained. 2. Operate the HTST system in forward-flow and activate the manual divert button until the raw pressure reaches zero (0) psi. Deactivate the manual divert button and observe the raw milk or milk product and pasteurized milk or milk product pressures. The pressure differential between raw and pasteurized milk or milk product in the regenerator should be maintained. 3. Re-seal the regulatory controls as necessary. Corrective Action: If the above described actions do not occur, or the necessary pressure differential between raw and pasteurized milk or milk product is not maintained, the assembly 268 and wiring of the HTST system must be immediately reviewed and the indicated deficiencies corrected or proper adjustments made. 5.6 RESPONSE TIME Apparatus: 1. Water, oil or other suitable media bath; 2. Suitable means of heating the media bath; and 3. Stopwatch. Method: Determine the elapsed time between the instant of the activation of the control mechanism at cut-out temperature on declining temperature and the instant the FDD takes the fully diverted-flow position. Procedure: 1. With the water, oil or suitable media bath at a temperature above cut-out temperature, allow the water, oil or other suitable media to cool gradually. The moment the cut-out mechanism is activated, start the watch. The moment the FDD takes the fully-diverted position, stop the watch. 2. Re-seal the regulatory controls as necessary and record the results. Corrective Action: If the response time exceeds one (1) second, immediate corrective action must be taken. 5.7 TIME DELAY INTERLOCK WITH TIMING PUMP Application: To all dual stem FDDs with a manual forward-flow switch. Apparatus: None Method: Determine that the device does not assume a manually induced forward-flow position, while the timing pump or any other flow-promoting device, which is capable of causing flow through the FDD is operating. Procedure: With the system operating in forward-flow, move the control switch to the "Inspect" position and observe that the following events automatically occur in sequence: 1. The FDD immediately moves to the diverted-flow position and the timing pump and all other flow-promoting devices, which are capable of causing flow through the FDD, are de-energized or in the case of separators, or downstream vacuum sources,are effectively valved-out of the system. Also, the isolation valves of any downstream vacuum source must be de-energized and in the closed position. 2. The FDD remains in the diverted-flow position while the timing pump and all other flow- promoting devices, which are capable of causing flow through the FDD are running down or in the case of a separator, or downstream vacuum sources, valving out. Also, the isolation valves of any downstream vacuum source must be de-energized and in the closed position. 3. The FDD may assume the forward-flow position only after the timing pump stops turning, and all other flow-promoting devices, which are capable of causing flow through the FDD have also stopped, or in the case of separators or downstream vacuum sources, have been effectively valved-out of the system. 4. Repeat the above procedure by moving the control switch to the “Cleaned-in-Place” (CIP) position. 5. Record the Test results and seal the control enclosure. Corrective Action: If the above sequence of events do not occur, either a timer adjustment or wiring change is required. 269 5.8 CIP TIME DELAY RELAY Application: To all continuous-flow pasteurizer systems in which it is desired to run the timing pump and/or other flow-promoting devices during the CIP cycle without the controls required during product processing. Criteria: When the mode switch on the FDD is moved from “Process” to “CIP”, the FDD shall move immediately to the diverted position. It shall remain in the diverted position for at least ten (10) minutes, with all public health controls required in “Process” mode functioning, before starting its normal cycling in the “CIP” mode. In HTST systems, the booster pump shall be de-energized during the ten (10) minute time delay. Apparatus: Stopwatch. Method: Determine that the set point on the time delay relay is equal to or greater than ten (10) minutes. Procedure: 1. Operate the pasteurizer in forward-flow, with the mode switch on the FDD in the “Process” position, using water above pasteurization temperature. For magnetic flow meter based timing systems, operate the system, at a flow-rate below the Flow-Alarm set point and above the Loss of-Signal-Alarm set point. NOTE: The appropriate temperature elements may be placed in a water or oil bath to simulate the normal pasteurization temperature of the holding tube as an alternative to heating the water in the system above the pasteurization temperature. 2. Move the mode switch on the FDD to the “CIP” position. The FDD should move immediately to the diverted position. Start the stopwatch when the FDD moves to the diverted position. Check all controls that are required to be in operation when the system is in the “Process” mode and in diverted-flow. For example, in HTST systems, the booster pump must stop running. Separators located between regenerator sections or on the pasteurized side of the system must be effectively valved-out and stuffer pumps for such separators must be de-energized. Any downstream vacuum source must be effectively valved-out.Also, the isolation valves of any downstream vacuum source must be de-energized and in the closed position. 3. Stop the stopwatch when the CIP timer times out. On most systems this is when the FDD moves to the forward position for its initial cycle in the “CIP” mode. At this time the system may be operated without the controls normally required during product processing. For example, the booster pump may start at this time. 4. Record the results. 5. Install and seal the enclosure over the time delay relay. Corrective Action: If the FDD does not remain in the diverted position for at least ten (10) minutes after the mode switch is moved from “Process” to “CIP”, increase the set point on the time delay relay and repeat this test procedure. All public health controls required when the system is in “Process” mode and in diverted-flow must be functional during these ten (10) minutes. If any of the public health controls are not functional during these ten (10) minutes, adjustments or repairs are needed. In HTST systems, if the booster pump runs at any time during the ten (10) minute delay, the booster pump wiring is in need of repair. 270. 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS; Proposal #:; 145. Committee:; Tech.. No Action; Passed as Submitted; Passed as Amended. COUNCIL ACTION; X Substitute. FINAL ACTION; X. C. Proposed Solution. Changes to be made on page(s):; 300; of the (X - one of the following):. X; 2007 PMO; 2005 EML. 2007 MMSR; 2400 Forms. 2007 Procedures; 2007 Constitution and Bylaws. PMO, APPENDIX I, PAGE 299: TEST 14. SETTING OF CONTROL SWITCHES FOR DIFFERENTIAL PRESSURE ACROSS THE INJECTOR Reference: Item 16p (B) and (E) Application: To all continuous flow pasteurizers and aseptic processing systems using direct contact injection heating. When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. Frequency: Upon installation; every three (3) months thereafter; and whenever the differential pressure controller seal is broken. Criteria: The pasteurizer or aseptic processor shall not operate in forward-flow unless the milk or milk product pressure drop across the injector is at least 69 kPa (10 psi). Apparatus: A sanitary pressure gauge and a pneumatic testing device described in Test 9.1 can be used for checking and adjusting the differential pressure controller. Method: Adjust the differential pressure switch to prevent forward-flow, unless the differential pressure across the injector is at least 69 kPa (10 psi). Test 14.1 Calibration of Steam Injector Controller; Reference Item 16pBE. Application: To all continuous flow pasteurizers and aseptic processing systems using direct contact heating. Frequency: Upon installation. Every three (3) months or whenever the steam injector controller is adjusted or repaired. Criteria: The pasteurizer or aseptic processor shall not operate in forward flow unless the milk or milk product pressure drop across the injector is at least 69 kpa (10 psi). Apparatus: A sanitary pressure gauge and pneumatic device described in test 9.1 can be used for checking and adjusting the accuracy of the required pressure sensors across the steam injector. Method: The pressure sensors are checked and calibrated for accuracy so as to prevent forward-flow unless the milk or milk product pressure drop across the injector is at least 69 kpa (10 psi). Procedure: Calibration of the Injector Differential Pressure Controller: 1. Loosen the process connection at both pressure sensors and wait allow for any liquid to drain through the loose connections. Both pointers, or digital displays, should shall be within 3.5 kPa (0.5 psi) of 0 kPa (0 psi). If not, adjust the pointer(s), or the digital display(s), to read 0 kPa (0 psi). - Remove both sensors from the processor and mount them in a tee, or connect them to a pneumatic testing device. Record any difference in the zero readings that may have occurred because of this change in elevation. Note the separation between the two (2) pointers or digital displays. The change ion elevation of the sensors will have caused some change in the zero reading. Attach the tee and both sensors to a pneumatic testing device described in Test 9.1 and adjust the air pressure to the normal operating pressure used at the injector. Make sure that the pointer or digital display reading separation is within 6.9 kPa (1 psi) of that observed before the pressure was applied. If not, the instrument requires adjustment or repair. 3. Adjust the air pressurer to the desired pressure range of the sensors. When the results are satisfactory, record the test results for the office record and proceed as directed below. - Observe the pressure sensor reading of both pointers or digital displays and record the test results for the office record. Corrective Action: If one or both pointers or digital displays are greater than 3.5 kpa( (0.5 psi) adjust and re-test and re-seal the regulatory controls. Setting of the Injector Differential Pressure Controller Switch: 1. Remove both pressure sensing elements from their original locations on the pasteurizer, or aseptic processor. Install a sanitary pressure gauge of known accuracy and Disconnect the sanitary pressure sensing element that is normally located after the steam injector from the pneumatic testing device and cap the resulting opening. Leave the pressure-sensing element, which is installed prior to the steam injection, on the pneumatic testing device. 2. Leave the other pressure sensing element open to the atmosphere, but at the same height as the sensing element connected to the pneumatic testing device. 3. Wire the test lamp in series with the differential controller micro-switch or use the method provided by the instrument manufacturer to monitor the cut-in signal. 4. Apply air pressure to the sensing element and determine, from the test lamp, the pressure gauge reading at the cut-in point of the differential pressure switch. 5. The differential pressure cut-in on the controller shall be at least 69 kPa (10 psi). If adjustment is necessary, refer to the manufacturer's instructions. 6. After adjustment, repeat the Test. 7. When the results are satisfactory, seal the instrument and record the results for the office record.. 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS; Proposal #:; 148. Committee:; SSCC. No Action; Passed as Submitted; Passed as Amended. COUNCIL ACTION; X. FINAL ACTION; X. C. Proposed Solution. Changes to be made on page(s):; 311 and FORM FDA 2359d; of the (X - one of the following):. X; 2007 PMO; 2007 EML. 2007 MMSR; 2400 Forms. 2007 Procedures; 2007 Constitution and Bylaws. Make the following changes to the 2007 PMO and FORM FDA 2359d: Strike out text to be deleted and underline text to be added. APPENDIX J. STANDARDS FOR THE FABRICATION OF SINGLE-SERVICE CONTAINERS AND CLOSURES FOR MILK AND MILK PRODUCTS SECTION E. CRITERIA FOR LISTING CERTIFIED SINGLE-SERVICE MANUFACTURERS IN THE IMS LIST Page 311: Historically, certification of manufacturers of single-service containers and related products has been for one (1) year. In addition, a ninety (90)-day grace period was provided for the transmission of the Report of Certification through the proper channels to Milk Safety Branch (HFS-626) to provide for the lag time for printing the in IMS List. The following criteria have been developed to allow Rating and/or Regulatory Agencies flexibility in evaluating and listing single-service manufacturing plants. Rating and/or Regulatory Agencies may choose from the following list of criteria for listing certified single-service manufacturers: 1. Single-service manufacturers that operate in conjunction with an IMS Listed milk plant may be listed for twenty-four (24) months, if the single-service plant is inspected at least quarterly, using Form FDA 2359c-Manufacturing Plant Inspection Report, and records of such inspections and all required tests are maintained by the Regulatory Agency. Provided that, single-service manufacturers that operate in conjunction with an IMS HACCP listed milk plant may be listed for twenty-four (24) months, if the single-service plant is integrated into the milk plant’s NCIMS HACCP system and if the single-service plant is inspected at the minimum milk plant audit frequency specified in Appendix K, using Form FDA 2359c-MANUFACTURING PLANT INSPECTION REPORT, and records of such inspections and all required tests are maintained by the Regulatory Agency. The permit for the milk plant shall also include the inspection of the single-service manufacturing areas. 2. Single-service manufacturers that operate in conjunction with an IMS Listed milk plant and are not inspected at least quarterly and/or are not included under a permit system may be optionally listed for twelve (12) months , plus a ninety (90)-day grace period after an evaluation. 3. Single-service manufacturers that operate as a separate entity may be listed for twenty-four (24) months, if the Regulatory Agency has a permit system and inspects the plant using Form FDA 2359c at least quarterly. All testing of containers and individual water supplies shall be under the direction of the Regulatory Agency and kept on file. 4. Single-service manufacturers that operate as a separate entity and are not inspected by Regulatory Agency personnel at least quarterly and/or do not have a permit system may be optionally listed for twelve (12) months, plus a ninety (90)-day grace period, after an evaluation. 5. Certification of single-service manufacturing plants may be valid for a period not to exceed one (1) or two (2) years from the earliest survey date, based on the criteria above. The expiration date is one (1) or two (2) years from the earliest survey date. In the case of a one (1) year certification with the earliest survey date of 6/15/2009, the expiration date would be 6/14/2010. FORM FDA 2359d-REPORT OF CERTIFICATION (Fabrication of Single-Service Containers and Closures for Milk and Milk Products) 7.c. EXPIRATION DATE* *EXPIRATION DATE This date is 15 or 24 months following the survey date. Certification of single-service manufacturing plants may be valid for a period not to exceed one (1) or two (2) years from the earliest survey date. In the case of a 1 year certification, a 90-day grace period is included to provide time for transmission of the completed Report of Certification (Form FDA 2359d). The expiration date is one (1) or two (2) years from the earliest survey date. NOTE: An effective date shall be the close of the Conference at which this Proposal is adopted.. 32nd NATIONAL CONFERENCE ON INTERSTATE MILK SHIPMENTS; Proposal #:; 150. Committee:; Tech.. No Action; Passed as Submitted; Passed as Amended. COUNCIL ACTION; X. FINAL ACTION; X. C. Proposed Solution. Changes to be made on page(s):; 345; of the (X - one of the following):. X; 2007 PMO; 2005 EML. 2007 MMSR; 2400 Forms. 2007 Procedures; 2007 Constitution and Bylaws. APPENDIX Q ITEM 14r. PROTECTION FROM CONTAMINATION The teat cups of the milking cluster need to be adequately shielded during the udder prepping process to assure that contaminants may not enter through the teat cup and get into the milk. AMIs are designed to automatically shift from milk to wash; therefore, adequate separation of milk and CIP solution shall be provided to minimize the risk of cross contamination of milk with cleaning and sanitizing solutions. A fail-safe valve system providing protection equivalent to an inter-wired block and-bleed, as referenced in Item15p.(B), shall be located as needed to prevent cross contamination. Separation shall be provided between, milk with abnormalities and milk intended for sale, and between cleaning/sanitizing solutions and milk intended for sale. A separation system shall be provided and located as needed to prevent cross contamination. Separation shall be provided between milk with abnormalities and milk intended for sale, and between cleaning/sanitizing solutions and milk intended for sale by approved systems providing protection equivalent to: - A fail-safe valve system equivalent to an inter-wired block and-bleed, as referenced in Item15p.(B) - A fail-safe instrumented air block system consisting of at least two (2) automatically controlled valves with an instrumented compartment between the valves where a blocking pressure can be created. (see figure 1 below) AMIs, which have a pipe into the wash vat that is continuously connected to the system, shall have a valving system that provides for an air break equal to the diameter of the wash line. Figure 1 sanitary air –> s injector check valve pressure sensor Inlet Valve High pressure Low pressure Low pressure Outlet Valve. |