ASEPTIC PILOT PROGRAM IMPLEMENTATION COMMITTEE

2011 NCIMS PROPOSAL

1/11/2011

GRADE “A” PASTEURIZED MILK ORDINANCE

2009 Revision

NOTE: Underlined text is proposed new wording and struck through text is wording that is proposed to be deleted.

Make the following changes to the TABLE OF CONTENTS on Pages viii, ix and xiii:

Page viii

STANDARDS FOR GRADE “A” RAW MILK FOR PASTEURIZATION, ULTRA-PASTEURIZATION OR ASEPTIC PROCESSING AND PACKAGING ………………

Page ix

STANDARDS FOR GRADE “A” PASTEURIZED, ULTRA-PASTEURIZED AND ASEPTICALLY PROCESSED AND PACKAGED MILK AND MILK PRODUCTS …

ITEM 16p. PASTEURIZATION AND ASEPTIC PROCESSING AND PACKAGING ...

ITEM 16p.(C). ASEPTIC PROCESSING SYSTEMS ……………………………………

ITEM 16p.(DC). PASTEURIZERS AND ASEPTIC PROCESSING SYTEMS

EMPLOYING REGENERATIVE HEATING …………………………………………..

MILK OR MILK PRODUCT-TO-MILK OR MILK PRODUCT REGENERATIVE HEATING ……...

MILK OR MILK PRODUCT-TO-WATER-TO-MILK OR MILK PRODUCT REGENERATIVE

HEATING  ……………………………………………………………………………………………….

ITEM 16p.(ED). PASTEURIZATION AND ASEPTIC PROCESSING RECORDS,

EQUIPMENT TESTS AND EXAMINATIONS ………………………………………..

Page xiii

Appendix Q.  Operation of Automatic Milking Installations for the Production of Grade “A” Raw Milk for Pasteurization, ULTRA-PASTEURIZATION OR ASEPTIC PROCESSING AND PACKAGING ………………………………………………………………………

APPENDIX S. ASEPTIC PROCESSING AND PACKAGING PROGRAM ………...

Make the following changes to TABLES on Page xv:

Page xv

Table 4. Equipment Tests – Batch Pasteurizers, and HTST, and HHST and Aseptic Processing Pasteurization Systems  ……………………………………………………

Make the following changes to SECTION 1. DEFINITIONS on Pages 1-11:

Page 11

b. Aseptic Processing AND PACKAGING: The term “Aseptic Processing and Packaging”, when used to describe a milk or milk product, means that the milk or milk product has been subjected to sufficient heat processing and packaged in a hermetically sealed container, to conform to the applicable requirements of 21 CFR Parts 108, 110 and 113 (Refer to the Reference in Appendix L.) and the provisions of Section 7, Item 16p of this Ordinance, and to maintain the commercial sterility of the product under normal non-refrigerated conditions.

C.	ASEPTIC PROCESSING AND PACKAGING SYSTEM (APPS):  For the purposes of this Ordinance, the Aseptic Processing and Packaging System in a milk plant is comprised of the processes and equipment used to process and package aseptic Grade "A" milk or milk products.  The APPS shall be regulated in accordance with the applicable requirements of 21 CFR Parts 108, 110 and 113.  The APPS shall begin at the constant level tank and end at the discharge of the packaging machine, provided that the Process Authority may provide written documentation which will clearly define additional processes or equipment that are considered critical to the commercial sterility of the product.

CD.  AUTOMATIC MILKING INSTALLATION (AMI): …..

Re-letter remaining definitions accordingly.

Page 5

V.	LOW-ACID ASEPTIC MILK AND MILK PRODUCTS: Milk or milk products having a water activity (aw) greater than 0.85 and a finished equilibrium pH greater than 4.6 and are regulated under 21 CFR Parts 108, 110 and 113.  Aseptically processed and packaged low-acid milk and milk products are stored under normal non-refrigerated conditions.  Excluded from this definition are low-acid milk and milk products that are labeled for storage under refrigerated conditions.

Re-letter remaining definitions accordingly.

VX.  MILK PLANT: A milk plant is any place, premises, or establishment where milk or milk products are collected, handled, processed, stored, pasteurized, ultra-pasteurized, aseptically processed and packaged, condensed, dried, packaged, or prepared for distribution.

Re-letter remaining Definitions accordingly and make specific Definition re-lettering citations throughout the PMO on Pages 6 (QQ to SS), 79, 81, 89-93, 100 (all FF to HH), 127 (II to KK), 176 and 359 (FF to HH and MM to OO).

Make the following changes to SECTION 4. LABELING on Pages 15 and 16:

Page 15

All bottles, containers and packages containing milk or milk products, except milk tank trucks, storage tanks and cans of raw milk from individual dairy farms, shall be conspicuously marked with:

1. 	The identity of the milk plant where pasteurized, ultra-pasteurized, aseptically processed and packaged, condensed and/or dried.

2. 	The words "keep refrigerated after opening" in the case of aseptically processed and packaged milk and milk products…..

Page 16

ADMINISTRATIVE PROCEDURES

IDENTITY LABELING: "Identity", as used in this Section, is defined as the name and address or permit number of the milk plant at which the pasteurization, ultra-pasteurization, aseptic processing and packaging, condensing and/or drying takes place.  It is recommended that the voluntary national uniform coding system for the identification of milk plants, at which milk and milk products are packaged, be adopted in order to provide a uniform system of codes throughout the country.

In cases where several milk plants are operated by one firm, the common firm name may be utilized on milk bottles, containers and packages.  Provided, that the location of the milk plant at which the contents were pasteurized, ultra-pasteurized, aseptically processed and packaged, condensed and/or dried is also shown, either directly or by a code.  This requirement is necessary in order to enable the Regulatory Agency to identify the source of the pasteurized, ultra-pasteurized, aseptically processed and packaged, condensed and/or dried milk or milk products.  The street address of the milk plant need not be shown when only one (1) milk plant of a given name is located within the municipality…..

MISLEADING LABELS: The Regulatory Agency shall not permit the use of any misleading marks, words or endorsements upon the label.  They may permit the use of registered trade designs or similar terms on the bottle cap or label, when in their opinion, they are not misleading and are not so used as to obscure the labeling required by this Ordinance.  For dry milk products, the outer bag must be preprinted "Grade "A" before filling. The use of super grade designations shall not be permitted.  However, this should not be construed as prohibiting the use of official grade designations awarded to dry milk products by the United States Department of Agriculture (usda).  Grade designations such as “Grade "AA" Pasteurized”, “Selected Grade "A" Pasteurized”, “Special Grade "A" Pasteurized”, etc., give the consumer the impression that such a grade is significantly safer than Grade “A”.  Such an implication is false, because the Ordinance requirements for Grade “A” pasteurized, ultra-pasteurized, or aseptically processed and packaged milk and milk products when properly enforced, will ensure that this grade of milk and milk products will be as safe as milk they can practically be made.  Descriptive labeling terms must not be used in conjunction with the Grade “A” designation or name of the milk or milk product and must not be false or misleading.

Make the following changes to SECTION 5. INSPECTION OF DAIRY FARMS AND MILK PLANTS on Pages 17-20:

Page 17

3. 	Inspect each milk plant and receiving station at least once every three (3) months, except provided that, for those milk plants and receiving stations that have HACCP Systems, which are regulated under the NCIMS HACCP Program, regulatory audits shall replace the regulatory inspections described in this Section. The requirements and minimum frequencies for these regulatory audits are specified in Appendix K.  Provided further, that regulatory inspections of a milk plant or portion of a milk plant that is IMS listed to produce aseptically processed and packaged milk or milk products shall be conducted by the State Regulatory Agency in accordance with this Ordinance at least once every six (6) months. (Refer to Appendix S.) The milk plant's APPS shall be inspected by FDA, or the State Regulatory Agency when designated by FDA, in accordance with the applicable requirements of 21 CFR Parts 108, 110 and 113 at a frequency determined by FDA. …..

Page 18

The Regulatory Agency shall take immediate action to prevent further movement of such milk or milk product until such violations of critical processing element(s) have been corrected.  Should correction of such critical processing element(s) not be accomplished immediately, the Regulatory Agency shall take prompt action to suspend the permit as provided for in Section 3 of this Ordinance.  Provided, that in the case of milk plants producing aseptically processed milk and milk products, when an inspection of the milk plant and its records reveal that the process used has been less than the required scheduled process, it shall be considered an imminent hazard to public health and the Regulatory Agency shall take immediate action to suspend the permit of the milk plant for the sale of aseptically processed milk and milk products in conformance with Section 3 of this Ordinance. …..

Page 19

ADMINISTRATIVE PROCEDURES

INSPECTION FREQUENCY: For the purposes of determining the inspection frequency for dairy farms, and transfer stations and milk plants or the portion of a milk plant that is IMS listed to produce aseptically processed and packaged milk or milk products, the interval shall include the designated six (6) month period plus the remaining days of the month in which the inspection is due.

For the purposes of determining the inspection frequency for all other milk plants and receiving stations the interval shall include the designated three (3) month period plus the remaining days of the month in which the inspection is due.

One (1) milk tank truck inspection every twelve (12) months,; or bulk milk hauler/sampler's or industry plant sampler's pickup and sampling procedures inspection each twenty-four (24) months,; or one (1) producer, transfer station, milk plant or portion of a milk plant that is IMS listed to produce aseptically processed and packaged milk or milk products or milk tank truck cleaning facility inspection every six (6) months,; or one (1) milk plant producing pasteurized, ultra-pasteurized, condensed or dried milk and milk products or receiving station inspection every three (3) months is not a desirable frequency, it is instead a legal minimum. …..

ENFORCEMENT PROCEDURES: This Section provides that a dairy farm, bulk milk hauler/sampler, milk tank truck, milk tank truck cleaning facility, milk plant, receiving station, transfer station or distributor, except those processing aseptically processed milk and milk products shall be subject to suspension of permit and/or court action if two (2) successive inspections disclose a violation of the same requirement. …..

Page 20

ENFORCEMENT PROCEDURES - ASEPTIC PROCESSING AND PACKAGING MILK PLANTS: Because aseptically processed milk and milk products are stored at room temperature and are not refrigerated after processing they must be considered an imminent hazard to public health whenever it is revealed by an inspection or a review of the processing records that the process is less than the required scheduled process and the products produced have not maintained their commercial sterility.  Prompt action by the Regulatory Agency to suspend the permit must be initiated in order to protect the public health.  The Regulatory Agency shall stop the sale of all under-processed milk or milk product and follow at least the minimum requirements of 21 CFR 113.89 before releasing any product.  (Refer to Appendix L.)  The State Regulatory Agency shall take appropriate regulatory action, in coordination with FDA when applicable, to assure that the Grade “A” aseptic milk plant and the Grade “A” aseptic milk and milk products meet the applicable requirements of this Ordinance.

Make the following changes to SECTION 6. THE EXAMINATION OF MILK AND MILK PRODUCTS on Pages 23-26:

Page 23

1. During any consecutive six (6) months, at least four (4) samples of raw milk for pasteurization, ultra-pasteurization, or aseptic processing and packaging shall be collected from each producer, in at least four (4) separate months, except when three (3) months show a month containing two (2) sampling dates separated by at least twenty (20) days.  These samples shall be obtained under the direction of the Regulatory Agency or shall be taken from each producer under the direction of the Regulatory Agency and delivered in accordance with this Section.

2. During any consecutive six (6) months, at least four (4) samples of raw milk for pasteurization, ultra-pasteurization or aseptic processing and packaging, shall be collected in at least four (4) separate months, except when three (3) months show a month containing two (2) sampling dates separated by at least twenty (20) days.  These samples shall be obtained by the Regulatory Agency, from each milk plant after receipt of the milk by the milk plant and prior to pasteurization, ultra-pasteurization or aseptic processing and packaging. …..

4. During any consecutive six (6) months, at least four (4) samples of pasteurized milk, ultra-pasteurized milk, flavored milk, flavored reduced fat or low fat milk, flavored nonfat (skim) milk, each fat level of reduced fat or low fat milk and each milk product defined in this Ordinance, shall be collected by the Regulatory Agency in at least four (4) separate months, except when three (3) months show a month containing two (2) sampling dates separated by at least twenty (20) days from every milk plant.  All pasteurized and (including Aseptically Processed and Ultra-Pasteurized) ultra-pasteurized milk and milk products required sampling and testing is to be done only when there are test methods available that are validated by FDA and accepted by the NCIMS.  Products with no validated and accepted methods are not required to be tested. Aseptically processed and packaged milk and milk products shall be exempt from the sampling and testing requirements of this Item …..

Required bacterial counts, somatic cell counts and cooling temperature checks shall be performed on raw milk for pasteurization, ultra-pasteurization or aseptic processing and packaging.  In addition, drug tests on each producer's milk shall be conducted at least four (4) times during any consecutive six (6) months.

Page 24

All pasteurized (including Aseptically Processed and Ultra-Pasteurized) ultra-pasteurized milk and milk products required sampling and testing to be done only when there are test methods available that are validated by FDA and accepted by the NCIMS, otherwise there would be no requirement for sampling.  Required bacterial counts, coliform counts, drug tests, phosphatase and cooling temperature determinations shall be performed on Grade "A" pasteurized and ultra-pasteurized milk and milk products defined in this Ordinance only when there are validated and accepted test methodology.

Note: When multiple samples of the same milk or milk products, except for aseptically processed and packaged milk and milk products, are collected from the same producer or processor from multiple tanks or silos on the same day, the laboratory results are averaged arithmetically by the Regulatory Agency and recorded as the official results for that day. This is applicable for bacterial (standard plate count and coliform), somatic cell count and temperature determinations only.

Whenever two (2) of the last four (4) consecutive bacterial counts (except those for aseptically processed milk and milk products), somatic cell count, coliform determinations, or cooling temperatures, taken on separate days, exceed the standard for the milk and/or milk products as defined in this Ordinance, the Regulatory Agency shall send a written notice thereof to the person concerned.  This notice shall be in effect as long as two (2) of the last four (4) consecutive samples exceed the standard.  An additional sample shall be taken within twenty-one (21) days of the sending of such notice, but not before the lapse of three (3) days.  Immediate suspension of permit, in accordance with Section 3, and/or court action shall be instituted whenever the standard is violated by three (3) of the last five (5) bacterial counts (except those for aseptically processed milk and milk products), somatic cell counts, coliform determinations or cooling temperatures. …..

Whenever a container or containers of aseptically processed milk or milk product is found to be non-sterile, due to under-processing, the Regulatory Agency shall consider this to be an imminent hazard to public health and shall suspend the permit of the milk plant for the sale of aseptically processed milk and milk products.  No aseptically processed milk and milk product shall be sold until it can be shown that the processes, equipment and procedures used are suitable for consistent production of a sterile product.  All products from the lot that were found to contain one (1) or more non-sterile units shall be recalled and disposed of as directed by the Regulatory Agency.

Samples shall be analyzed at an appropriate official or officially designated laboratory.  All sampling procedures, including the use of approved in-line samplers and aseptic samplers for milk tank trucks, and required laboratory examinations shall be in substantial compliance with the most current edition of Standard Methods for the Examination of Dairy Products (SMEDP) of the American Public Health Association, and the most current edition of Official Methods of Analysis of AOAC INTERNATIONAL (OMA).  Such procedures, including the certification of sample collectors and examinations shall be evaluated in accordance with the Procedures.  Aseptically processed milk and milk products packaged in hermetically sealed containers shall be tested in accordance with FDA's Bacteriological Analytical Manual (BAM).  ….

Page 25

Assays of milk and milk products as defined in this Ordinance, including aseptically processed and packaged milk and milk products, to which vitamin(s) A and/or D have been added for fortification purposes, shall be made at least annually in a laboratory, which has been accredited by FDA and which is acceptable to the Regulatory Agency, using test methods acceptable to FDA or other official methodologies, which gives statistically equivalent results to the FDA methods.  Vitamin testing laboratories are accredited if they have one (1) or more certified analysts and meet the quality control requirements of the program established by FDA.  Laboratory accreditation and analyst certification parameters are specified in the Evaluation of Milk Laboratories (EML) manual. ….

ADMINISTRATIVE PROCEDURES

ENFORCEMENT PROCEDURES: All violations of bacteria, coliform, confirmed somatic cell counts and cooling temperature standards should be followed promptly by inspection to determine and correct the cause.  (Refer to Appendix E. Examples of Three (3)-out-of-Five (5) Compliance Enforcement Procedures)

Aseptically processed milk and milk products packaged in hermetically sealed containers are exempt from the refrigerated storage requirements of this Ordinance.  Therefore, whenever a breakdown in the processing or packaging of these products occurs an imminent hazard to public health exists.  Prompt action is needed by the Regulatory Agency.  Milk plants aseptically processing milk and milk products in hermetically sealed containers should be encouraged to perform bacterial and other quality tests on each lot of aseptically processed milk and milk product produced in order to ascertain that these products have been properly processed and have not been rendered non-sterile after aseptic processing and packaging.  The Regulatory Agency may utilize industry records, of each lot of aseptically processed milk and milk products, to determine when lots can be released for sale after a violation of the bacterial standards has existed.

Make the following changes to SECTION 7. STANDARDS FOR GRADE “A” MILK AND MILK PRODUCTS on Pages 28-31:

Page 28

All Grade “A” raw milk or milk products for pasteurization, or ultra-pasteurization, or aseptic processing and packaging and all Grade "A" pasteurized, ultra-pasteurized or aseptically processed and packaged milk and milk products, shall be produced, processed, manufactured and pasteurized, ultra-pasteurized, or aseptically processed and packaged to conform to the following chemical, physical, bacteriological and temperature standards and the sanitation requirements of this Section.

No process or manipulation other than pasteurization, ultra-pasteurization or aseptic processing and packaging; processing methods integral therewith; and appropriate refrigeration shall be applied to milk and milk products for the purpose of removing or deactivating microorganisms, provided that filtration and/or bactofugation processes are performed in the milk plant in which the milk or milk product is pasteurized, ultra-pasteurized or aseptically processed and packaged.  Provided, that in the bulk shipment of cream, nonfat (skim) milk or reduced fat or lowfat milk, the heating of the raw milk, one time, to temperatures greater than 52ºC (125ºF) but less than 72ºC (161ºF), for separation purposes, is permitted when the resulting bulk shipment(s) of cream, nonfat (skim) milk or reduced fat or lowfat milk are labeled heat-treated.  In the case of heat-treated cream, the cream may be further heated to less than 75ºC (166ºF) in a continuing heating process and immediately cooled to 7ºC (45ºF) or less when necessary for enzyme deactivation (such as lipase reduction) for a functional reason. …..

Pages 29-30

| Table 1. Chemical, Physical, Bacteriological, and Temperature Standards |  |  |
| --- | --- | --- |
| GRADE “A” RAW MILK AND MILK PRODUCTS FOR PASTEURIZATION, ULTRA-PASTEURIZATION OR ASEPTIC PROCESSING AND PACKAGING | Temperature............ | Cooled to 10ºC (50ºF) or less within four (4) hours or less, of the commencement of the first milking, and to 7°C (45ºF) or less within two (2) hours after the completion of milking. Provided, that the blend temperature after the first milking and subsequent milkings does not exceed 10ºC (50ºF). NOTE: Milk sample submitted for testing cooled and maintained at 0ºC (32ºF) to 4.4ºC (40ºF), where sample temperature is >4.4ºC (40ºF), but ≤7.0ºC (45oF) and less than three (3) hours after collection has not increased in temperature. |
|  | Bacterial Limits........................ | Individual producer milk not to exceed 100,000 per mL prior to commingling with other producer milk. Not to exceed 300,000 per mL as commingled milk prior to pasteurization. NOTE: Tested in conjunction with the drug residue/inhibitory substance test. |
|  | Drugs....................... | No positive results on drug residue detection methods as referenced in Section 6 - Laboratory Techniques. |
|  | Somatic Cell Count*.. | Individual producer milk not to exceed 750,000 per mL. |
| GRADE “A” PASTEURIZED MILK AND MILK PRODUCTS AND BULK SHIPPED HEAT-TREATED MILK PRODUCTS | Temperature............ | Cooled to 7ºC (45ºF) or less and maintained thereat. NOTE: Milk sample submitted for testing cooled and maintained at 0ºC (32ºF) to 4.4ºC (40ºF), where sample temperature is >4.4ºC (40ºF), but ≤7.0ºC (45oF) and less than three (3) hours after collection has not increased in temperature. |
|  | Bacterial Limits**…... | Not to exceed 20,000 per mL, or gm.*** NOTE: Tested in conjunction with the drug residue/inhibitory substance test. |
|  | Coliform****............ | Not to exceed 10 per mL. Provided, that in the case of bulk milk transport tank shipments, shall not exceed 100 per mL. NOTE: Tested in conjunction with the drug residue/inhibitory substance test. |
|  | Phosphatase*****..... | Less than 350 milliunits/L for fluid products and other milk products by approved electronic phosphatase procedures. |
|  | Drugs**.................….. | No positive results on drug residue detection methods as referenced in Section 6 - Laboratory Techniques which have been found to be acceptable for use with pasteurized and heat-treated milk and milk products. |
| GRADE "A" PASTEURIZED CONCENTRATED (CONDENSED) MILK AND MILK PRODUCTS | Temperature………. | Cooled to 7oC (45oF) or less and maintained thereat unless drying is commenced immediately after condensing. |
|  | Coliform…………… | Not to exceed 10 per gram. Provided, that in the case of bulk milk transport tank shipments shall not exceed 100 per gram. |
| GRADE “A” Ultra- Pasteurized Milk and Milk Products | Temperature............... | Cooled to 7ºC (45ºF) or less and maintained thereat. |
|  | Bacterial Limits**…. | Not to exceed 20,000 per mL, or gm.*** |
|  | Coliform****............ | Not to exceed 10 per mL. Provided, that in the case of bulk milk transport tank shipments, shall not exceed 100 per mL. |
|  | Phosphatase*****..... | Phosphatase testing of Ultra-Pasteurized milks is not required. |
|  | Drugs**.................… | There are no validated and accepted drug residue tests for Ultra-Pasteurized Milk and Milk Products |
| GRADE “A” ASEPTICALLY PROCESSED AND PACKAGED MILK AND MILK PRODUCTS | Temperature............... | None. |
|  | Bacterial Limits......... | Refer to 21 CFR 113. 3(e)(1)****** |
|  | Drugs**...................... | There are no validated and accepted drug residue tests for Aseptically Processed Milk and Milk Products. |
| GRADE "A" NONFAT DRY MILK | Butterfat…………… Moisture…………… Titratable Acidity…. Solubility Index…… Bacterial Estimate… Coliform……………. Scorched Particles disc B……………… | No More Than: 1.25% 4.00% 0.15% 1.25mL. 30,000 per gram 10 per gram 15.0 per gram |
| GRADE "A" WHEY FOR CONDENSING AND/OR DRYING | Temperature……… | Maintained at a temperature of 45oF (7oC) or less, or 57oC (135oF) or greater, except for acid-type whey with a titratable acidity of 0.40% or above, or a pH of 4.6 or below. |
| GRADE "A" PASTEURIZED CONDENSED WHEY AND WHEY PRODUCTS | Temperature………. | Cooled to 10oC (50oF) or less during crystallization, within 72 hours of condensing. |
|  | Coliform Limit……. | Not to exceed 10 per gram. |
| GRADE "A" DRY WHEY, GRADE "A" DRY WHEY PRODUCTS, GRADE "A" DRY BUTTERMILK, AND GRADE "A" DRY BUTTERMILK PRODUCTS | Coliform Limit……. | Not to exceed 10 per gram. |

*  Goat Milk 1,500,000/mL

** Not applicable to acidified or cultured products, eggnog and flavored (non-chocolate) milk and milk products.

Page 31

*** Results of the analysis of dairy products which are weighed in order to be analyzed will be reported in # per gm. (Refer to the current edition of the SMEDP)

**** Not applicable to bulk shipped heat-treated milk products.

***** Not applicable to bulk shipped heat-treated milk products; UP products that have been thermally processed at or above 1380C (2800F) for at least two (2) seconds to produce a product which has an extended shelf life (ESL) under refrigerated conditions; and condensed products.

****** 21 CFR 113.3(e)(1) contains the definition of “COMMERCIAL STERILITY”.

STANDARDS FOR GRADE “A” RAW MILK FOR PASTEURIZATION, ULTRA-PASTEURIZATION OR ASEPTIC PROCESSING AND PACKAGING

Make the following changes to SECTION 7. STANDARDS FOR GRADE “A” PASTEURIZED, ULTRA-PASTEURIZED AND ASEPTICALLY PROCESSED MILK AND MILK PRODUCTS on Page 55:

Page 55

STANDARDS FOR GRADE “A” PASTEURIZED, ULTRA-PASTEURIZED AND ASEPTICALLY PROCESSED AND PACKAGED MILK AND MILK PRODUCTS

Milk plants shall comply with all Items of this Section. Provided, in the case of milk plants or portions of milk plants that are IMS Listed to produce aseptically processed and packaged milk or milk products, the APPS, as defined by this Ordinance, shall be exempt from Items 7p, 10p, 11p, 12p, 13p, 15p, 16p, 17p, 18p, and 19p of this Ordinance and shall comply with the applicable portions of 21 CFR Parts 108, 110 and 113.  Those Items, contained within the APPS, shall be inspected by FDA or a State Regulatory Agency, when designated by FDA. …..

Milk plants that have HACCP Systems, which are regulated under the NCIMS HACCP Program, shall comply with all of the requirements of Item 16p. Pasteurization and Aseptic Processing and Packaging of this Ordinance, and pasteurization shall be managed as a CCP as described in Appendix H. VIII- Milk and Milk Product Continuous-flow (HTST and HHST) Pasteurization---CCP Model HACCP Plan Summary; and Milk and Milk Product Vat (Batch) Pasteurization---CCP Model HACCP Plan Summary.

Make the following changes to ITEM 1p. FLOORS - CONSTRUCTION on Page 56:

Page 56

ADMINISTRATIVE PROCEDURES

3. The floors are provided with trapped drains.  Cold-storage rooms used for storing milk and milk products need not be provided with floor drains when the floors are sloped to drain to one or more exits.  Storage rooms for dry ingredients, dry packaged milk or milk products, aseptically processed and packaged milk or milk products and/or packaging materials need not be provided with drains. …..

Make the following changes to ITEM 2p. WALLS AND CEILINGS - CONSTRUCTION on Page 57:

Page 57

NOTE:  Refer to Item 11p for requirements for walls for drying chambers.  Storage rooms used for the storage of packaged dry milk or milk products and aseptically processed and packaged milk or milk products are exempt from the ceiling requirements of this Item.

Make the following changes to ITEM 5p. SEPARATE ROOMS on Page 58:

Page 58

4.	The fabrication of containers and closures for milk and milk products, except for aseptically processed and packaged milk and milk products that are fabricated within the APPS.

Make the following changes to ITEM 11p. CONSTRUCTION AND REPAIR OF CONTAINERS AND EQUIPMENT on Page 66:

Page 66

ADMINISTRATIVE PROCEDURES

12.  Provided that all paper, plastics, foil, adhesives, and other components of containers and closures used in the packaging of milk or milk products that have been aseptically processed are governed under the applicable provisions of 21 CFR Parts 110 and 113 and shall not be subject to this Section.

Make the following changes to ITEM 12p. CLEANING AND SANITIZATION OF CONTAINERS AND EQUIPMENT on Pages 66 and 70:

Page 66

The product-contact surfaces of all multi-use containers, utensils and equipment used in the transportation, processing, condensing, drying, packaging, handling, and storage of milk or milk products shall be effectively cleaned and shall be sanitized before each use.  Provided, that cloth-collector systems used on dryers shall be cleaned and sanitized or purged at intervals and by methods recommended by the manufacturer and approved by the Regulatory Agency.   Provided further, that piping, equipment and containers used to process, conduct or package aseptically processed milk and milk products, beyond the final heat treatment process, shall be sterilized before any aseptically processed milk or milk product is packaged and shall be re-sterilized whenever any non-sterile product has contaminated it.

Page 70

ADMINISTRATIVE PROCEDURES

5.	All multi-use containers, utensils and equipment are sanitized before use, employing one or a combination of the methods prescribed under Item 11r.  Additionally, for milk plants that condense or dry milk or milk products the following methods are acceptable, or any other method, which has been demonstrated to be equally efficient:

a. 	Exposure to an enclosed jet of steam for not less than 1 minute.

b. 	Exposure to hot air at a temperature of at least 83oC (180oF) for at least twenty (20) minutes as measured by an acceptable indicating thermometer located in the coldest zone.

Assembled equipment must be sanitized prior to each day's run, unless FDA and the Regulatory Agency have reviewed and accepted information supporting the sanitizing of multi-use containers, utensils and equipment at frequencies extending beyond one (1) day.  Tests to determine the efficiency of sanitization should be made by the Regulatory Agency at intervals sufficient to satisfy the Regulatory Agency that the sanitization process is effective.  Provided, that all piping, equipment and containers used to conduct, process or package aseptically processed milk and milk products, beyond the final heat treatment process, shall be sterilized by heat, chemical sterilant(s) or other appropriate treatment before use and resterilized whenever it has been contaminated by nonsterile product. ….

Make the following changes to ITEM 15p. PROTECTION FROM CONTAMINATION on Pages 78 and 79:

15p.(B)

Page 78

c. 	In the case of aseptically processed and higher-heat-shorter-time (HHST) pasteurized milk and milk products that are processed and the equipment cleaned and/or chemically sanitized above the atmospheric boiling point of the milk or milk product or cleaning and/or sanitizing solutions, the required separation between pipe lines and equipment, used to contain or conduct milk and milk products, and tanks or circuits containing cleaning and/or chemical sanitizing solutions, may be accomplished using an alarmed steam block(s), located between the milk and milk product and cleaning and/or chemical sanitizing solutions if: …..

(4)	The temperature sensor is integrated with automatic controls, such that when there is milk or milk products on one (1) side of the steam block and cleaning and/or chemical sanitizing solutions on the other side of the steam block, and the temperature sensor in the steam trace detects a temperature that indicates that liquid, rather than steam, is present in the steam trace, the cleaning pump will be de-energized, and when needed to prevent solution pressure on the steam block, the cleaning and/or chemical sanitizing solution are automatically drained away from the steam block. Except that:

Page 79

i) In in systems where the cleaning and/or sanitizing solution is circulated by the timing pump, that pump may continue to operate during an alarmed condition, provided a legal flow-diversion device (FDD) is used to divert the cleaning and/or chemical sanitizing solution flow away from the steam block.

ii) In aseptic processing systems that are not equipped with a legal FDD and where the cleaning and/or sanitizing solution is circulated by the timing pump of the aseptic processing system, that pump may continue to operate during an alarmed condition, provided there are at least two (2) instrumented steam blocks between the milk and milk product and the cleaning and/or chemical sanitizing solutions and at least one (1) of the blocks remains uncompromised. …..

NOTE: The valve arrangement(s) described in this Section shall not be used to separate raw products, dairy, non-dairy or water, from pasteurized milk or milk products.  Provided that, nothing in this Section shall be construed as barring any other means to separate milk and milk product from cleaning/sanitizing solution in systems, which have been recognized by FDA and in the case of aseptic processing equipment, by the Processing Authority, to be equally effective and which are approved by the Regulatory Agency.

Make the following changes to ITEM 16p. PASTEURIZATION AND ASEPTIC PROCESSING on Page 81:

Page 81

ITEM 16p.  PASTEURIZATION AND ASEPTIC PROCESSING AND PACKAGING

Pasteurization shall be performed as defined in Section 1, Definition FF HH and Item 16p of this Ordinance.  Aseptic processing and packaging shall be performed in accordance with the applicable requirements of 21 CFR Parts 113 , 108, 110 and 113 the Administrative Procedures of Item 16p, sub-items (C), (D) and (E) of this Section. (Refer to Appendix L.) …..

A note of caution is in order.  Although pasteurization destroys the organisms, it does not destroy the toxins that may be formed in milk and milk products when certain staphylococci are present, as from udder infections, and when the milk or milk product is not properly refrigerated before pasteurization.  Such toxins may cause severe illness.  Aseptic processing and packaging has also been conclusively demonstrated to be effective in preventing outbreaks from milkborne pathogens. Numerous studies and observations clearly prove that the food value of milk is not significantly impaired by pasteurization. …..

Make the following changes to ITEM 16p. PASTEURIZATION AND ASEPTIC PROCESSING on Page 84:

ADMINSTRATIVE PROCEDURES

Page 84

6. The design and operation of pasteurization equipment and all appurtenances thereto shall comply with the applicable specifications and operational procedures of Subitems (A), (B), (DC) and (ED).

Make the following changes to ITEM 16p.(A) BATCH PASTEURIZATION on Page 88:

ADMINISTRATIVE PROCEDURES

Page 88

5. RECORDING CHARTS:

All recording thermometer charts shall comply with all the applicable requirements of Item 16p(ED)1.a.

Make the following changes to ITEM 16p.(B) HIGH-TEMPERATURE-SHORT-TIME (HTST) CONTINUOUS-FLOW PASTEURIZATION on Page 92:

ADMINISTRATIVE PROCEDURES

Page 92

e.	Indicating and Recording Thermometers: …..

(3)	The recorder/controller charts shall comply with the applicable provisions of Item 16p(ED)1.a.

Make the following changes to ITEM 16p.(C) ASEPTIC PROCESSING SYSTEMS on Pages 95-98:

Pages 95-98

ITEM 16p.(C) ASEPTIC PROCESSING SYSTEMS

PUBLIC HEALTH REASON

Aseptically processed milk and milk products are being packaged in hermetically sealed containers and stored for long periods of time under non-refrigerated conditions.  These conditions are favorable to the growth of many types of bacteria, including pathogenic, toxin producing and spoilage organisms.  Because of this, every precaution must be taken to ensure that the chosen heat process, for the particular milk or milk product, destroys all viable organisms and their spores. The subsequent handling, packaging and storage processes do not provide an opportunity for recontamination of the milk or milk product.  The selected process must conform to the acceptable requirements for low acid canned foods.

ADMINISTRATIVE PROCEDURES

This Item is deemed to be satisfied when:

The design and operation of aseptic processing systems comply with the applicable specifications and operational procedures of Item 16p, sub-items (C), (D) and (E). Provided, that nothing shall be construed as barring any other aseptic processing system which have been recognized by FDA to be equally effective and which is approved by the Regulatory Agency.

1.	INDICATING THERMOMETERS AND RECORDER/CONTROLLER INSTRUMENTS:

All indicating thermometers, recorder/controller instruments and devices, used in connection with aseptic processing systems, used for the aseptic processing of milk or milk products shall comply with the applicable specifications set forth in Appendix H.

2.  ASEPTIC PROCESSING EQUIPMENT:

a.	Temperature Indicating Device: Each aseptic processing system shall be equipped with at least one (1) mercury-in-glass thermometer or an equivalent temperature-indicating device.

b. 	Temperature Recorder/Controller: An accurate temperature recorder/controller shall be installed in the milk or milk product at the holding tube outlet and before the inlet to the cooler or regenerator.  The following requirements shall be met with respect to the instrumentation of the temperature recorder/controller:

(1)	The temperature recorder/controller shall be set and sealed so that during milk or milk product processing the forward-flow of milk or milk product cannot start unless the temperature at the controller sensor is above the required temperature for the milk or milk product and the process used, nor continue during descending temperatures when the temperature is below the required temperature.

The seal shall be applied by the Regulatory Agency after testing and shall not be removed without immediately notifying the Regulatory Agency.  The system shall be so designed that no milk or milk product can be bypassed around the controller sensor, which shall not be removed from its proper position during the processing of aseptic milk and milk products.

(2)	Additional temperature-controllers and timers shall be interwired with the thermal-limit controller, and the control system shall be set and sealed so that forward-flow of milk or milk product cannot start until all product-contact surfaces between the holding tube and FDD have been held at or above the required sterilization temperature, continuously and simultaneously for at least the required sterilization time.  The control system shall also be set and sealed so that forward-flow cannot continue when the temperature of the milk or milk product in the holding tube is below the required temperature.  The seal shall be applied by the Regulatory Agency after being tested and shall not be removed without immediately notifying the Regulatory Agency.  The system shall be so designed that no milk or milk product can be bypassed around the control sensors, which shall not be removed from their proper position during the processing of aseptic milk and milk products.

(3)	Manual switches for the control of pumps, homogenizers or other devices that produce flow through the holding tube, shall be wired so that the circuit is completed only when the milk or milk product is above the required temperature for the milk or milk product and the process used, or when the FDD is in the fully diverted position.

c.  	Timing Pump:

(1)	A positive displacement type timing pump located upstream from the holding tube, or a magnetic flow meter based timing system, which complies with the specifications as outlined in Appendix H, shall be operated to maintain the required rate of milk or milk product flow.  The motor of the timing pump shall be connected by means of a common drive shaft, or by means of gears, pulleys or a variable-speed drive, with the gear box, the pulley box or the setting of the variable speed protected in such a manner that the hold time cannot be shortened without detection by the Regulatory Agency.  This shall be accomplished by the application of a suitable seal(s) after being tested by the Regulatory Agency and such seal(s) shall not be broken without immediately notifying the Regulatory Agency.  This provision shall apply to all homogenizers used as timing pumps.  Variable speed drives, used in connection with the timing pump, shall be so constructed that wearing or stretching of the belt results in a slowdown, rather than a speedup, of the pump.  The timing pump shall be of the positive-displacement type or shall comply with the specifications for magnetic flow meter based timing systems.

(2)	The holding time shall be taken to mean the flow time of the fastest particle of milk or milk product throughout the holding tube section, i.e., that portion of the system that is outside of the influence of the heating medium; and slopes continuously upward in the downstream direction; and is located upstream from the FDD.

d.  	Milk or Milk Product Holding Tube:

(1)	The milk or milk product holding tube shall be designed to give continuous holding of every particle of milk or milk product for at least the minimum holding time specified in the scheduled process.  The holding tube shall be designed, so that no portion of the holding tube between the milk or milk product inlet and the milk or milk product outlet can be heated. In addition, it must be sloped upward at least 2.1 centimeters per meter (0.25 of an inch per foot).  Supports for holding tubes shall be provided to maintain all parts of the holding tubes in a fixed position, free from any lateral or vertical movement.

(2)	No device shall be permitted for short-circuiting a portion of the holding tube to compensate for changes in rate of milk or milk product flow.  Holding tubes shall be installed so that sections of pipe cannot be left out, resulting in a shortened holding time.  The holding time for the processes must be determined from the pumping rate, rather than by the salt conductivity test.

(3)	The holding tube length must be such that the fastest flowing particle of any milk or milk product will not traverse the holding tube in less than the required holding time.

NOTE:  With the direct addition of steam, the holding time is reduced because the milk or milk product volume increases as the steam condenses to water during heating.  This surplus water is evaporated as the aseptically processed milk or milk product is cooled in the vacuum chamber.  For example, with a 66ºC (120ºF) increase by steam injection, which is probably the maximum temperature rise that will be used, a volume increase of twelve percent (12%) will occur in the holding tube.  The measurement of the average flow rate at the discharge of the aseptic processor does not reflect this volume increase in the holding tube.  However, this volume increase, i.e., holding time decrease, must be considered in the calculations.

(4)	An aseptic processing system which can operate with milk or milk product in forward-flow mode, with less than 518 kPa (75 psig) pressure in the holding tube shall be equipped with a pressure limit indicator/pressure switch in the holding tube to assure that the heated milk or milk product remains in the liquid phase.  In systems that do not have a vacuum chamber between the holding tube and the aseptic milk or milk product side of the regenerator, this can be established by verifying that the aseptic processing equipment cannot operate in forward-flow with less than 518 kPa (75 psig) pressure on the aseptically processed side of the regenerator. (Refer to Appendix I., Test 9).  The pressure limit indicator/pressure switch must be interwired so that the FDD, milk or milk product divert system, milk or milk product divert valve or other acceptable control system will move to the divert position, if the milk or milk product pressure falls below a prescribed value.  The instrument must be set at a pressure 69 kPa (10 psi) above the boiling pressure of the milk or milk product at its maximum temperature in the holding tube.  If this pressure is too low, the resultant vaporization in the holding tube will substantially reduce residence times.

(5)	With the steam injection process, a differential pressure limit indicator, across the injector, is needed to ensure adequate isolation of the injection chamber.  The instrument must have a differential pressure switch so that the FDD will move to the divert position if the pressure drop across the injector falls below 69 kPa (10 psi).

e.  	Heating by Direct Addition of Steam:  Steam injection is an inherently unstable process; accordingly, when steam is injected into a fluid, condensation of the steam may not be completed inside the injector unless the proper design criteria are used.  Lack of complete condensation inside the injector would cause temperature variations in the holding tube, which could lead to some milk or milk product particles being processed below filed process temperature.  When culinary steam is injected directly into milk or milk products, as the means of terminal heating to achieve aseptic processing temperature, the steam injector shall be designed, installed and operated to comply with the following or equally satisfactory specifications:

(1)	The milk or milk product and steam flows must be isolated from pressure fluctuations inside the injection chamber.  One (1) method of isolation is to insert supplementary orifices on the milk or milk product inlet and the heated milk or milk product outlet of each injector.  The two (2) supplementary orifices must be sized for at least a 69 kPa (10 psi) milk or milk product pressure drop across the injector during a simulation of normal operations.  Excessive vibrations, pressure fluctuations or erratic noise levels indicate an unstable steam injection system and a need to check the isolation of the injection chamber.

(2)	The process should be as free as possible of non-condensable gases that may evolve from the milk or milk product or be carried in the steam supply.  Any two (2) phase flow, caused by the non-condensable gases, would displace the milk or milk product in the holding tube, resulting in reduced residence times.  In addition, these gases in the steam supply may also markedly alter the condensation mechanism at the point of injection.  Accordingly, the steam boiler shall be supplied with a de-aerator.  The de-aerator will aid in keeping the milk or milk product in the holding tube as free as possible of non-condensable gases.

f. 	Prevention of Milk or Milk Product Adulteration with Added Water:

(1)	When culinary steam is introduced directly into the milk or milk product, automatic means, i.e., stand-alone and/or PLC-based ratio control system, shall be provided to maintain a proper temperature differential between incoming and outgoing milk or milk products to preclude dilution with water.

(2)	Where a water feed line is connected to a vacuum condenser and the vacuum condenser is not separated from the vacuum chamber by a physical barrier, means shall be provided to preclude the back-up and overflow of water from the vacuum condenser into the vacuum chamber.  This provision may be satisfied by the use of a safety shutoff valve, located on the water feed line to the vacuum condenser that is automatically actuated by a control that shuts off the in-flowing water.  This valve may be actuated by water, air or electricity and shall be so designed that failure of the primary motivating power will automatically stop the flow of water into the vacuum condenser.

g. 	FDD: All FDDs used in continuous aseptic process systems shall comply with Item 16p(B)2.b. or equally satisfactory specifications.

Make the following changes to ITEM 16p.(D) PASTEURIZERS AND ASEPTIC PROCESSING SYSTYEMS EMPLOYING REGENERATIVE HEATING on Pages 98-102:

Page 98

ITEM 16p.(DC) PASTEURIZERS AND ASEPTIC PROCESSING SYSTYEMS EMPLOYING REGENERATIVE HEATING

ADMINISTRATIVE PROCEDURES

This Item is deemed satisfied when: …..

Page 99

MILK OR MILK PRODUCT-TO-MILK OR MILK PRODUCT REGENERATIVE HEATING

Pasteurizers and aseptic processing systems employing milk or milk product-to-milk or milk product regenerative heating with both sides closed to the atmosphere shall comply with the following or equally satisfactory specifications:

1.	Regenerators shall be constructed, installed and operated so that pasteurized or aseptic milk or milk product in the regenerator will automatically be under greater pressure than raw milk or milk product in the regenerator at all times.

2.	The pasteurized or aseptic milk or milk product, between its outlet from the regenerator and the nearest point downstream open to the atmosphere, shall rise to a vertical elevation of 30.5 centimeters (12 inches) above the highest raw milk or milk product level, downstream from the constant-level tank, and shall be open to the atmosphere at this or a higher elevation. …..

4.	No pump or flow-promoting device which can affect the proper pressure relationships within the regenerator shall be located between the pasteurized or aseptic milk or milk product outlet from the regenerator and the nearest downstream point open to the atmosphere.

5.	No pump shall be located between the raw milk or milk product inlet to the regenerator and the constant-level tank, unless it is designed and installed to operate only when milk or milk product is flowing through the pasteurized or aseptic milk or milk product side of the regenerator and when the pressure of the pasteurized or aseptic milk or milk product is higher than the maximum pressure produced by the pump.  This may be accomplished by wiring the booster pump so that it cannot operate unless:

a.	The timing pump is in operation;

b.	The FDD is in forward-flow position; and

c.	The pasteurized or aseptic milk or milk product pressure exceeds, by at least 6.9 kPa (1 psi), the maximum pressure developed by the booster pump.  Pressure gauges shall be installed at the raw milk or milk product inlet to the regenerator and the pasteurized or aseptic milk or milk product outlet of the regenerator or the outlet of the cooler.  The accuracy of these required pressure gauges shall be checked, by the Regulatory Agency, on installation; quarterly thereafter; and following repair or adjustment. …..

9.	When vacuum equipment is located downstream from the FDD, means shall be provided to prevent the lowering of the pasteurized or aseptic milk or milk product level in the regenerator during periods of diverted-flow or shutdown.  An effective vacuum breaker, plus an automatic means of preventing a negative pressure, shall be installed in the line between the vacuum chamber and the pasteurized or aseptic milk or milk product inlet to the regenerator.

Page 100

10.	In the case of pasteurization systems, with the FDD located downstream from the regenerator and/or cooler section, the requirements of paragraphs (2), (3), (5), (7) and (8) of this Section may be eliminated.  Provided, that a differential pressure controller is used to monitor the highest pressure in the raw milk or milk product side of the regenerator and the lowest pressure in the pasteurized side of the regenerator, and the controller is interlocked with the FDD and is set and sealed so that whenever improper pressures occur in the regenerator, forward-flow of milk or milk product is automatically prevented and will not start again until all milk or milk product-contact surfaces between the holding tube and FDD have been held at or above the required pasteurization temperature, continuously and simultaneously for at least the required pasteurization time as defined in Definition FF of this Ordinance.

In the case of aseptic processing systems used for producing aseptic milk and milk products, there shall be an accurate differential pressure recorder-controller installed on the regenerator.  The scale divisions shall not exceed 13.8 kPa (2 psi) on the working scale of not more than 138 kPa (20 psi) per 2.54 centimeters (1 inch).  The controller shall be tested for accuracy against a known accurate standard pressure indicator upon installation; at least once every three (3) months of operation thereafter; or more frequently if necessary, to ensure its accuracy.  One (1) pressure sensor shall be installed at the aseptic milk or milk product regenerator outlet and the other pressure sensor shall be installed at the raw milk or milk product regenerator inlet.

11.	When culinary steam is introduced directly into milk or milk product to achieve pasteurization or aseptic processing temperature, and vacuum equipment is located downstream from the holding tube, the requirement that a vacuum breaker be installed at the inlet to the pasteurized or aseptic side of the regenerator may be eliminated.  Provided, that the differential pressure controller is installed and wired to control the FDD as described in paragraph 10 of this Section. …..

MILK OR MILK PRODUCT-TO-WATER-TO-MILK OR MILK PRODUCT REGENERATIVE HEATING

Page 101

Option II. Milk or milk product-to-water-to-milk or milk product regenerators may also be constructed, installed and operated such that the pasteurized or aseptic milk or milk product in the regenerator will be under greater pressure than the heat-transfer-medium in the pasteurized or aseptic milk or milk product side of the regenerator:

1.	A differential pressure controller shall be used to monitor pressures of the pasteurized milk or milk product and the heat-transfer-medium.

2.	In the case of aseptic processing systems, a differential pressure-recorder shall be used to monitor pressures of the aseptic milk or milk product and the heat-transfer-medium.

3.	In either case, one One (1) pressure sensor shall be installed at the pasteurized or aseptic milk or milk product outlet of the regenerator and the other pressure sensor shall be installed at the heat-transfer-medium inlet of the pasteurized or aseptic milk or milk product side of the regenerator.  This controller or recorder-controller shall divert the FDD whenever the lowest pressure of pasteurized or aseptic milk or milk product in the regenerator fails to exceed the highest pressure of the heat-transfer-medium in the pasteurized or aseptic milk or milk product side of the regenerator by at least 6.9 kPa (1 psi).  Forward-flow of milk or milk product shall be automatically prevented until all milk or milk product-contact surfaces between the holding tube and the FDD have been held at or above the required pasteurization or sterilization temperature continuously and simultaneously for at least the pasteurization or sterilization time.

42.	The heat-transfer-medium pump shall be wired so that it cannot operate unless the timing pump is in operation.

Make the following changes to ITEM 16p.(E) PASTEURIZATION AND ASEPTIC PROCESSING RECORDS, EQUIPMENT TESTS AND EXAMINATIONS on Pages 102-105:

Page 102

ITEM 16p.(ED) PASTEURIZATION AND ASEPTIC PROCESSING RECORDS, EQUIPMENT TESTS AND EXAMINATIONS

1. 	PASTEURIZATION AND ASEPTIC PROCESSING RECORDS:

All temperature and flow rate pasteurization recording charts or alternative records, acceptable to FDA, in place of charts shall be preserved for a period of three (3) months.  Provided, that all records and recording charts for aseptic milk and milk product systems shall be retained for a period of three (3) years.  The use of such charts shall not exceed the time limit for which they are designed.  Overlapping of recorded data shall be a violation of this Item.  The following information shall be entered on the charts or other records acceptable to FDA in place of charts as applicable: …..

Page 103

c.	Continuous-Flow Pasteurizers or Aseptic Processing Equipment with Magnetic Flow Meter Based Timing Systems: Flow rate recording charts shall be capable of continuously recording flow at the flow alarm set point and at least 19 liters (5 gallons) per minute higher than the high flow alarm setting.  Flow rate recording charts shall contain all the information specified in Subitem a. above, except (3), (4), (5), and (6), and (7) and in addition, shall include the following: …..

d.	Aseptic Processing Systems: Recording thermometer charts shall contain all the information specified in Subitem a. above, except (4) and (5).  In addition these records shall include Subitem c. above, if applicable, and the following:

(1) A continuous record of the time during which the FDD, valve or system is in the forward-flow position;

(2)	 A continuous record of applicable regenerator pressures;

(3) Not later than one (1) working day after the actual process, and before shipment or release for distribution, a representative of the milk plant management, who is qualified by suitable training or experience, shall review all processing and production records for completeness and to ensure that the milk or milk product received the schedule process.  The records, including the recording thermometer chart(s), shall be signed or initialed and dated by the reviewer; and

(4) Number (6) from above shall also be recorded immediately after a chart has been changed.

ed.	Electronic Data Collection, Storage and Reporting: Electronic collection, storage and reporting of required pasteurization and aseptic processing records, with or without hard copy printouts, may be acceptable, provided, the electronically generated records are readily available at the milk plant for review by the Regulatory Agency and meet the criteria of this Section and Appendix H., V.

2. 	EQUIPMENT TESTS AND EXAMINATIONS: ….

Page 104

In the case of milk plants with HACCP Plans regulated under the NCIMS HACCP Program, pasteurization and aseptic processing equipment may be tested and sealed by industry personnel acceptable to the Regulatory Agency, if the following conditions are met:

a.	Test results for Pasteurization and Aseptic Processing Equipment Testing shall be recorded on a similar document for all milk plants. (Refer to the reference in Appendix M. for an example.)

b.	Industry personnel conducting the Pasteurization and Aseptic Processing Equipment Testing must be adequately trained and must be able to demonstrate an acceptable understanding and ability to conduct these tests to the Regulatory Agency. …..

c.	Pasteurization and Aseptic Processing Equipment Tests shall be conducted at a frequency not less than the requirements of this Ordinance.  Industry shall have responsibility for the performance of all required tests.  At least each six (6) months the Regulatory Agency shall physically supervise these tests. Regulatory supervised tests shall include the semi-annual HTST and HHST tests. These six (6) month tests should be performed at a time that is mutually convenient to all parties. Because these tests are required to support a CCP, the industry is responsible for conducting these tests even in the absence of the regulatory official.

Page 105

d.	Upon initial installation or extensive modification of any pasteurization and aseptic processing equipment, tests shall be physically supervised or conducted by the Regulatory Agency. …..

f.	During an audit, the auditor may conduct any or all of the Pasteurization or Aseptic Processing Equipment Tests.  The auditor should, through a combination of physical examination of the equipment and a records review, satisfy themselves that the equipment is properly installed and operated.

Make the following changes to TABLE 4. EQUIPMENT TESTS – BATCH, HTST, HHST and ASEPTIC PROCESSING SYSTEMS on Page 106:

| Table 4. Equipment Tests – Batch Pasteurizers, and HTST, and HHST and Aseptic Processing Pasteurization Systems (Refer to Appendix I.) |  |  |
| --- | --- | --- |
| 1. | Vat, HTST, and HHST, Aseptic indicating and airspace thermometers | Temperature accuracy |
| 2. | Vat, HTST, and HHST, Aseptic recording thermometer | Temperature accuracy |
| 3. | Vat, HTST, and HHST, Aseptic recording thermometer | Time accuracy |
| 4. | Vat, HTST, and HHST, Aseptic indicating and recording thermometer | Recording vs. Indicating thermometer |
| 5.1 | HTST, and HHST FDD | Leakage pass FDD |
| 5.2 | HTST, and HHST FDD | FDD freedom of movement |
| 5.3 | HTST, and HHST FDD | Device assembly (single stem) |
| 5.4 | HTST, and HHST FDD | Device assembly (dual stem) |
| 5.5 | HTST FDD | Manual diversion |
| 5.6 | HTST, and HHST FDD | Response time |
| 5.7 | HTST, and HHST FDD | Time delay (inspect) |
| 5.8 | HTST, and HHST FDD | Time delay (CIP) |
| 5.9 | HTST FDD | Time delay (leak-detect flush) |
| 6. | Vat leak-protector valve(s) | Leakage |
| 7. | HTST indicating thermometers | Response time |
| 8. | HTST recording thermometers | Response time |
| 9.1 | HTST pressure switches | Regenerator pressures |
| 9.2.1 | HTST, and HHST, Aseptic differential pressure controllers | Calibration |
| 9.2.2 | HTST differential pressure controllers | Regenerator pressure |
| 9.2.3 | HTST*, and HHST and Aseptic differential pressure controllers | Regenerator pressure |
| 9.3.1 | HTST booster pump/FDD | Inter-wiring check |
| 9.3.2 | HTST booster pump/timing pump | Inter-wiring check |
| 10.1 | HTST FDD | Temperature cut-in/cut-out |
| 10.2 | HTST*, and HHST FDD, Aseptic divert system (indirect heat) | Temperature cut-in/cut-out |
| 10.3 | HTST*, and HHST FDD, Aseptic divert system (direct heat) | Temperature cut-in/cut-out |
| 11.1 | HTST holding tubes/timing pumps (except magnetic flow meter based timing systems-MFMBTS) | Holding time |
| 11.2.a | HTST holding tubes/ MFMBTS | Holding time |
| 11.2.b | HTST, and HHST, Aseptic MFMBTS | Flow alarm |
| 11.2.c | HTST, and HHST, Aseptic MFMBTS | Loss of signal/low flow |
| 11.2.d | HTST magnetic flow meter based timing systems | Flow rate cut-in/cut-out |
| 11.2.e | HTST magnetic flow meter based timing systems | Time delay |
| 11.2.f | All magnetic flow meter based timing systems | High flow alarm response time |
| 11.3 | HHST holding tubes (indirect heat) | Holding time |
| 11.4 | HHST holding tubes (direct injection heat) | Holding time |
| 11.5 | HHST holding tubes (direct infusion heat) | Holding time |
| 12.1 | HTST*, and HHST, Aseptic systems (indirect heating) | Sequence logic |
| 12.2 | HTST*, and HHST, Aseptic systems (direct heating) | Sequence logic |
| 13. | HHST, Aseptic systems | Pressure in the holding tube |
| 14. | HTST*, and HHST, Aseptic systems using (direct injection heating) | Pressure differential across injector |
| 15. | HTST, and HHST, Aseptic (all electronic controls) | Electro-Magnetic Interference |

* For HTST systems with the FDD located downstream of the regenerator and/or cooler section.

Make the following changes to ITEM 17p. COOLING OF MILK AND MILK PRODUCTS on Page 109:

Administrative procedures

Page 109

6.	Each refrigerated room in which pasteurized milk and milk products are stored, except aseptically processed milk and milk products, is equipped with an indicating thermometer that complies with the applicable specifications of Appendix H.  Such thermometer shall be located in the warmest zone of the refrigerated room.

Make the following changes to ITEM 18p. BOTTLING, PACKAGING AND CONTAINER FILLING on Page 113:

Administrative procedures

Page 113

12.	In the case of aseptic processing systems, the milk and milk product shall be aseptically filled into sterilized containers and hermetically sealed in conformance with the applicable requirements of 21 CFR Part 113.

Make the following changes to SECTION 8. ANIMAL HEALTH on Pages 117-118:

Page 117

1.	All milk for pasteurization, ultra-pasteurization or aseptic processing and packaging shall be from herds in Areas which have a Modified Accredited Advanced Tuberculosis (TB) status or higher as determined by the USDA.  Provided, that in an Area which fails to maintain such status, any herd shall have been accredited by said Department as tuberculosis free, or shall have passed an annual tuberculosis test, or the Area shall have established a tuberculosis testing protocol for livestock that assures tuberculosis protection and surveillance of the dairy industry within the Area and that it is approved by FDA, USDA and the Regulatory Agency. …..

2.	All milk for pasteurization, ultra-pasteurization or aseptic processing and packaging shall be from herds under a brucellosis eradication program, which meets one (1) of the following conditions: …..

Page 118

3.	Goat, sheep, water buffalo, or any other hooved mammal milk for pasteurization, ultra-pasteurization or aseptic processing and packaging, defined under this Ordinance, shall be from a herd or flock that: …..

Make the following changes to SECTION 9. MILK AND MILK PRODUCTS WHICH MAY BE SOLD on Page 120:

Page 120

From and after twelve (12) months from the date on which this Ordinance is adopted, only Grade “A” pasteurized, ultra-pasteurized, or aseptically processed and packaged milk and milk products shall be sold to the final consumer, to restaurants, soda fountains, grocery stores or similar establishments.  Provided, only Grade "A" milk and milk products shall be sold to milk plants for use in the commercial preparation of Grade "A' milk and milk products.  Provided further, that in an emergency, the sale of pasteurized, ultra-pasteurization or aseptic processed and packaged milk and milk products, which have not been graded, or the grade of which is unknown, may be authorized by the Regulatory Agency, in which case, such milk and milk products shall be labeled "ungraded".

Make the following changes to SECTION 11. MILK AND MILK PRODUCTS from points beyond the limits of routine inspection on Pages 121-123:

Page 121

Milk and milk products, from points beyond the limits of routine inspection of the ... of... or its jurisdiction, shall be sold in…, 1 or its jurisdiction provided they are produced and pasteurized, ultra-pasteurized, aseptically processed and packaged, concentrated (condensed) or dried under regulations which are substantially equivalent to this Ordinance and have been awarded acceptable Milk Sanitation Compliance and Enforcement Ratings; or have been awarded a an satisfactory acceptable HACCP listing, under the NCIMS HACCP Program as specified in Appendix K. of this Ordinance,; or are from a country that PHS/FDA has determined, after conferring with NCIMS, to have in place a public health regulatory program and government oversight of that program that have an equivalent effect on the safety of  regulated milk and/or milk products.

ADMINISTRATIVE PROCEDURES

The Regulatory Agency should accept, without their actual physical inspection, supplies of milk and milk products from an area or an individual shipper not under their routine inspection.  Provided, that: …..

Page 122

2.	After receipt, pasteurized, ultra-pasteurized, aseptically processed and packaged, concentrated (condensed) or dried milk and milk products shall comply with Sections 2, 4 and 10.

NOTE:  Raw, and pasteurized and ultra-pasteurized milk and milk products beyond the limits of routine inspection shall be sampled as the Regulatory Agency requires. …..

11.	Aseptically processed and packaged milk and milk products in Definition X of this Ordinance shall be considered to be Grade "A" milk or milk products.  The source sources(s) of the milk and milk products used to produce aseptically processed and packaged milk and milk products shall be IMS listed and the aseptic raw milk receiving area/aseptic raw milk receiving station of the milk plant where the aseptic milk and milk products are processed and packaged shall be IMS listed. Aseptically processed and packaged milk and milk products shall be labeled "Grade "A"" and meet Section 4 labeling requirements of the PMO.  The milk plant or portion of the milk plant that is producing aseptically processed and packaged milk and milk products shall be awarded a Milk Sanitation Compliance Rating of a at least ninety percent (90%) and a satisfactory ASEPTIC MILK PLANT REGULATORY AGENCY REVIEW REPORT or a satisfactory HACCP listing by a SRO trained under the NCIMS Aseptic Pilot Program and label its milk and milk products as “Grade “A””. an Enforcement Rating equal to the local supply, or equal to ninety percent (90%) or higher, or if the Enforcement Rating is below ninety percent (90%) on a rating, a re-rating must occur within (6) months of this rating.  Both the Milk Sanitation Compliance and Enforcement Ratings must be equal to ninety percent (90%) or higher on the re-rating or the supply is considered in violation of this Section.  In the case of HACCP/Aseptic listings, an acceptable HACCP listing by a SRO is required. For milk plants that produce aseptically processed and packaged Grade “A” milk and/or milk products, prior to the milk plant participating in the NCIMS Aseptic Processing and Packaging Program, or the Aseptic Pilot Program, the State’s regulatory and rating personnel shall have completed a training course that is acceptable to the NCIMS and FDA addressing the procedures for conducting regulatory inspections and ratings under the NCIMS Aseptic Processing and Packaging Program or Aseptic Pilot Program.  The NCIMS Aseptic Pilot Program addressing aseptically processed and packaged acidified and fermented high acid milk and milk products regulated under 21 CFR Parts 108, 110, and/or 114 will expire on December 31, 2011 2013, unless extended by future conference action.

Make the following changes to SECTION 13. PERSONNEL HEALTH on Page 123:

Page 123

No persons affected with any disease capable of being transmitted to others through the contamination of food shall work at a milk plant in any capacity which brings them into direct contact with pasteurized, ultra-pasteurized or aseptically processed and packaged milk or milk products or which brings them into direct contact with associated pasteurized or aseptically processed and packaged milk or milk product-contact surfaces.

Make the following changes to SECTION 14. PROCEDURE WHEN INFECTION OR HIGH RISK OF INFECTION IS DISCOVERED on Pages 124 and 125:

Page 124

When a person who may have handled pasteurized, ultra-pasteurized or aseptically processed and packaged milk or milk products or pasteurized, ultra-pasteurized or aseptically processed and packaged milk or milk product-contact surfaces meets one (1) or more of the conditions specified in the Administrative Procedures of Section 13, the Milk Regulatory Agency is authorized to require any or all of the following measures: …..

Page 125

NOTE: Persons at risk who decline to be examined may be reassigned to duties where they will not be required to handle pasteurized, ultra-pasteurized or aseptically processed and packaged milk or milk products and associated milk or milk product-contact surfaces.

Make the following changes to APPENDIX H. PASTEURIZATION EQUIPMENT AND PROCEDURES AND OTHER EQUIPMENT on Pages 216 and 217:

- HTST PASTEURIZATION

Page 216

HTST PASTEURIZERS EMPLOYING MILK or milk product-TO-MILK or milk product REGENERATORS WITH BOTH SIDES CLOSED TO THE ATMOSPHERE

Item 16p(DC), of Section 7 establishes standards for regenerators.  These standards insure that the raw milk or milk product will always be under less pressure than pasteurized milk or milk product in order to prevent contamination of the pasteurized milk or milk product in the event flaws should develop in the metal or joints separating it from the raw milk or milk product.  An explanation of regenerator specifications is given below.

During normal operation, i.e., while the timing pump is operating, raw milk or milk product will be drawn through the regenerator at sub-atmospheric pressure. The pasteurized milk or milk product in the milk or milk product-to-milk or milk product regenerator will be above atmospheric pressure.  The required pressure differential will be assured when there is no flow-promoting device downstream from the pasteurized milk or milk product side of the regenerator to draw the pasteurized milk or milk product through the regenerator, and the pasteurized milk or milk product downstream from the regenerator rises to at least 30.5 centimeters (12 inches) elevation above the highest raw milk or milk product level downstream from the constant-level tank, and is open to the atmosphere at this or a higher elevation, as required in Item 16p(DC), Administrative Procedures #2.

During a shutdown, i.e., when the timing pump stops, the raw milk or milk product in the regenerator will be retained under suction, except this suction may be gradually relieved by possible entrance of air drawn through the regenerator plate gaskets from the higher outside atmospheric pressure.  With a free draining regenerator, as required under Item 16p(DC), Administrative Procedures #8, the raw milk or milk product level in the regenerator may drop slowly, depending on the tightness of the gaskets, ultimately falling below the level of the plates to the milk or milk product level in the constant-level tank.  However, under these conditions, as long as any raw milk or milk product remains in the regenerator, it will be at sub-atmospheric pressure.

During shutdown, the pasteurized milk or milk product in the regenerator is maintained at atmospheric pressure or above by meeting the elevation requirement of Item 16p(DC), Administrative Procedures #2.  Pressure greater than atmospheric is maintained when the level of pasteurized milk or milk product is at or above the required elevation and loss of pressure, due to suction, is prevented by prohibiting a downstream pump.

Any backflow of milk or milk product through the FDD would lower the pasteurized milk or milk product level, during pump shutdowns, thus tending to reduce the pressure on the pasteurized milk or milk product side of the regenerator.  A FDD cannot be relied upon to prevent backflow in such instances, because during the first few minutes following a pump shutdown, the milk or milk product is still at a sufficiently high temperature to keep the FDD in the forward-flow position. Compliance with the provisions of Item 16p(DC), Administrative Procedures #2 and #3; however, will insure a proper pressure differential in the regenerator.

Page 217

At the beginning of a run, from the time raw milk or milk product or water is drawn through the regenerator, until the pasteurized milk or milk product or water has risen to the elevation specified in Item 16p(DC), Administrative Procedures #2, the pasteurized milk or milk product side of the regenerator is at atmospheric pressure or higher.  Even if the timing pump should stop during this period, the pressure on the pasteurized milk or milk product side of the regenerator will be greater than the sub-atmospheric pressure on the raw milk or milk product side.  This will be assured by compliance with Item 16p(DC), Administrative Procedures #2 and #3, as long as any raw milk or milk product remains in the regenerator.

When a raw milk or milk product booster pump is incorporated into the HTST pasteurization system, Item 16p(DC), Administrative Procedures #5 requires, in part, that automatic means shall be provided to assure, at all times, the required pressure differential between raw and pasteurized milk or milk product in the regenerator, before the booster pump can operate.

Make the following changes to APPENDIX H. PASTEURIZATION EQUIPMENT AND PROCEDURES AND OTHER EQUIPMENT on Pages 219 and 220:

- HTST PASTEURIZATION

Page 219

PRESSURE RELIEF VALVES LOCATED WITHIN HTST PASTEURIZATION SYSTEMS

OPTION I: …..

c. 	The system is designed and operated so that loss of pressure from the pasteurized side of the regenerator cannot occur if the system flow-promoting devices stop while the FDD is in the forward-flow position.  A system not protected against this potential pressure loss is considered a violation of Item 16p(DC) of this Ordinance

Page 220

Option II. The pressure relief valve is spring-loaded and plumbed so that it cannot be opened or forced open in any mode, “Product”, “CIP” or “Inspect”, without the assistance of pressure from the liquid flowing through the system. In this case, a leaking pressure relief valve can cause an unacceptable loss of pressure in the pasteurized side of the regenerator if the system flow-promoting devices stop while the FDD is in the forward-flow position. This is considered a violation of Item 16p(DC) of this Ordinance.  Any leakage from this pressure relief valve must be readily visible.  This may be accomplished by opening the pressure relief valve vent directly to the floor or by providing sanitary piping from the pressure relief valve vent to the constant-level tank.  If the later option is utilized, the piping shall be properly sloped to assure drainage to the constant-level tank and shall be provided with a properly located and installed sight-glass.

2.	Downstream from the Holding Tube: The pressures in the pasteurized side of the regenerator must be protected from falling within 6.9 kPa (1 psi) of the pressures in the raw side of the regenerator at all times, including during shut down. A relief valve and line on the pasteurized side of the FDD can meet this criterion if: …..

c. 	The pressure relief valve is spring-loaded and plumbed so that it cannot be opened or forced open in any mode, “Product”, “CIP” or “Inspect”, without the assistance of pressure from the liquid flowing through the system. In this case, a leaking pressure relief valve can cause an unacceptable loss of pressure in the pasteurized side of the regenerator during a shut down and is considered a violation of Item 16p(DC) of this Ordinance.  Any leakage from this pressure relief valve must be readily visible.  This may be accomplished by opening the pressure relief valve vent directly to the floor or by providing sanitary piping from the pressure relief valve vent to the constant-level tank.  If the later option is utilized, the piping shall be properly sloped to assure drainage to the constant-level tank and shall be provided with a properly located and installed sight-glass.

Make the following changes to APPENDIX H. PASTEURIZATION EQUIPMENT AND PROCEDURES AND OTHER EQUIPMENT on Page 222:

magnetic flow meter based timing systems for WITHIN htst CONTINUOUS FLOW pasteurizers SYSTEMS

Components: …..

Page 222

10.	All systems shall be designed, installed and operated so that all applicable tests required by Section 7, Item 16p(ED) can be performed by the Regulatory Agency, at the frequency specified.  (Refer to Appendix I.) Where adjustment or changes can be made to these devices or controls, appropriate seals shall be applied by the Regulatory Agency after testing, so that changes cannot be made without detection.

Make the following changes to APPENDIX H. PASTEURIZATION EQUIPMENT AND PROCEDURES AND OTHER EQUIPMENT on Pages 251-253:

V.  CRITERIA FOR THE EVALUATION OF Electronic Data Collection, Storage and Reporting

page 251

criteria

The following criteria are to be used for the evaluation of electronic collection, storage and recording or reporting of any information required within Items 12p and 16p(ED). of Section 7 of this Ordinance.

Page 252

5. 	In the case of pasteurization and aseptic processing records, data shall be stored no less than every five (5) seconds for each required variable. Any event required to be recorded in manual reporting, such as a divert condition; will be recorded no matter how short the duration. Provisions will be made to allow operators to report additional events electronically, such as a record of unusual occurrences. The data for the reporting system shall be backed up at least once every twenty-four (24) hours. Alternatively, the final reports may be stored and backed up at least once every twenty-four (24) hours. …..

Page 253

Note: While electronic and computerized systems can furnish a wide range of process validation and anomaly reporting, these criteria only require appended reporting of data loss that affects the reports that are required to comply with this Appendix and Items 12p and 16p(ED) or other required reporting contained in this Ordinance. …..

Make the following changes to APPENDIX I. PASTEURIZATION EQUIPMENT AND CONTROLS - TEST on Pages 273-276, 281-291, 296, 297, 299, and 305-311:

Page 273

II.  TEST PROCEDURES

TEST 1.

INDICATING THERMOMETERS - TEMPERATURE ACCURACY

Reference:  Item 16p (A), (B), (C) and (ED)

Application: To all indicating thermometers used for the measurement of milk or milk product temperature during pasteurization or aseptic processing, including airspace thermometers. …..

Criteria: Within ± 0.25ºC (± 0.5ºF) for pasteurization and aseptic processing ultra-pasteurization thermometers and ± 0.5ºC (± 1ºF) for airspace thermometers, in a specified scale range.  Provided, that on batch pasteurizers used solely for thirty (30) minute pasteurization of milk or milk products at temperatures above 71°C (160°F), indicating thermometers shall be accurate to within ± 0.5°C  (± 1°F). …..

Procedure:

1.  Prepare a quantity of water, oil or other suitable media in a bath, by raising the temperature of the media to within 2ºC (3ºF) of the appropriate pasteurization, or airspace temperature, or aseptic processing temperature.

Page 274

TEST 2.

RECORDING THERMOMETERS - TEMPERATURE ACCURACY

Reference:  Item 16p (A), (B), (C) and (ED)

Application: To all mercury-actuated recording and recorder-controller thermometers controllers used to record milk or milk product temperatures during pasteurization or aseptic processing. …..

NOTE:  When this Test is performed on mercury-actuated recorder-controllers used with HHST pasteurization or aseptic processing systems that operate at or above the boiling point of water, an oil or other suitable media bath shall be substituted for the processing (operating) temperature water mentioned in Procedures 1, 4, 5, 6, and 7 as well as the boiling water mentioned in Procedures 2, 3 and 5. The temperature of the oil bath that is used in place of the boiling water shall be above the normal operating range but below the highest temperature division on the chart. …..

Procedure:

2.  	Prepare a second media bath by heating to the boiling point, or in the case of HHST or aseptic pasteurization systems, to a temperature above the normal operating range but below the highest temperature division on the chart, and maintain temperature. Prepare a third container with melting ice.  Place all media baths within working distance of the temperature-sensing element(s).

3.	Immerse the recording thermometer sensing element into the boiling water, or in the case of HHST or aseptic processing pasteurization systems into the media bath described above, for not less than five (5) minutes. …..

Page 275

TEST 3.

RECORDING THERMOMETERS - TIME ACCURACY

Reference:  Item 16p (A), (B), (C) and (ED)

Application:  To all recording and recorder-controller thermometers used to record the time of pasteurization or aseptic processing. …..

Criteria: The recorded time of pasteurization or aseptic processing shall not exceed the true elapsed time. …..

Page 276

TEST 4.

RECORDING THERMOMETERS - CHECK AGAINST

INDICATING THERMOMETERS

Reference:  Item 16p (A), (B), (C) and (ED)

Application: To all recording and recorder-controller thermometers used to record milk or milk product temperatures during pasteurization or aseptic processing.

Frequency:  Upon installation and at least once each three (3) months by the Regulatory Agency, or HACCP qualified industry person, acceptable to the Regulatory Agency, qualified under Item 16p(ED)2; and daily by the milk plant operator. …..

Method:  This test requires only that the reading of the recording thermometer or the recorder-controller thermometer be compared with the indicating thermometer at a time when both are exposed to milk or milk product at a stabilized pasteurization or aseptic processing temperature.

Procedure:

1. 	While the indicating and recording temperatures are stabilized at or above the minimum legal pasteurization or aseptic processing temperature, read the indicating thermometer. …..

TEST 5.

FDD - PROPER ASSEMBLY AND FUNCTION

Reference:  Item 16p (B), (C) and (ED)

Application:  Test 5 (parts 1 through 9) does not apply to aseptic processing divert systems, valves or other acceptable controls which may be used in place of a FDD. Parts 1 to 4 and 6 to 8 apply to all FDDs used with continuous-flow pasteurizers. Parts 5 and 9 apply only to FDDs used with HTST pasteurizers. …..

Page 281

TEST 6.

LEAK-PROTECTOR VALVE

Reference:  Item 16p (A) and (ED) …..

Page 282

TEST 7.

INDICATING THERMOMETERS ON PIPELINES -

THERMOMETRIC RESPONSE

Reference:  Item 16p (B) and (ED) …..

Page 283

TEST 8.

RECORDER/CONTROLLER - THERMOMETRIC RESPONSE

Reference:  Item 16p (B) and (ED) …..

Page 284

TEST 9.

REGENERATOR PRESSURE CONTROLS

Reference:  Item 16p (DC) and (ED) …..

Page 285

9.2 DIFFERENTIAL PRESSURE CONTROLLER

Application: Test 9.2.1 applies to all differential pressure controllers used to control the operation of booster pumps on HTST pasteurization systems or used to control the operation of FDDs on HHST and HTST Pasteurization pasteurization systems with the FDD located downstream of the pasteurized regenerator and/or final cooler and aseptic processing systems. …..

Test 9.2.3 applies to the testing of continuous flow pasteurization systems in which the differential pressure controller is used to control the operation of the FDD. Test 9.2.3 also applies to aseptic processing systems in which the differential pressure controller is used to control the FDD, milk or milk product divert system, milk or milk product divert valve or other acceptable control system. …..

Page 286

Criteria:  The booster pump shall not operate, or the pasteurizer shall not operate in forward- flow, unless the milk or milk product pressure in the pasteurized side of the regenerator is at least 6.9 kPa (1 psi) greater than the milk or milk product pressure in the raw side of the regenerator.  When the differential pressure controller is used to control the FDD on HHST or aseptic processing pasteurization systems, and improper pressure occurs in the regenerator, the FDD shall move to the diverted-flow position and remain in diverted-flow until the proper pressures are re-established in the regenerator and all milk or milk product-contact surfaces between the holding tube and FDD have been held at or above the required pasteurization or aseptic processing temperature, continuously and simultaneously for at least the required time. …..

Method:  The differential pressure switch is checked and adjusted to prevent the operation of the booster pump, or prevent forward-flow, unless the milk or milk product pressure in the pasteurized, or aseptic, side of the regenerator is at least 6.9 kPa (1 psi) greater than the pressure in the raw side of the regenerator.

Page 287

9.2.3 INTERWIRING OF THE PRESSURE DIFFERENTIAL CONTROLLER WITH THE FDD IN AN HHST CONTINUOUS FLOW PASTEURIZATION SYSTEM; OR AN ACCEPTABLE ALTERNATIVE DEVICE, OR SYSTEM IN ASEPTIC PROCESSING EQUIPMENT

Application: …..

2. 	To all differential pressure controllers used to control the operation of FDDs, milk or milk product divert systems, or milk or milk product divert valve(s) or other acceptable control systems used in aseptic processing equipment. …..

Method:  The differential pressure switch is checked and adjusted to prevent forward-flow, unless the milk or milk product pressure in the pasteurized side of the regenerator is at least 6.9 kPa (1 psi) greater than the pressure in the raw milk or milk product side of the regenerator. In the case of milk or milk product-to-water-to-milk or milk product regenerators, protected on the pasteurized or aseptic side, the “water side” of the regenerator shall be considered to be the "raw product side" for purposes of this Test.

Procedure: …..

3.  	Adjust the pressure on the pressure switch sensors to their normal operating pressures, with the pasteurized or aseptic pressure at least 14 kPa (2 psi) higher than the raw product pressure.

a.	The test lamp should be lit. If not, increase the pasteurized or aseptic pressure, or lower the raw product pressure, until the test light is lit.

b.	Gradually lower the pasteurized or aseptic side, or raise the raw product pressure until the test light turns off.

c.	The test light should turn off when the pasteurized or aseptic pressure is at least 14 kPa (2 psi) higher than the raw product pressure.

d.	Note the differential pressure at the point the light turns off.

e.	Gradually raise the pasteurized or aseptic pressure, or lower the raw product pressure, until the test light turns on.

Page 288

f.	The test light should not turn on until the pasteurized or aseptic pressure is at least 14 kPa (2 psi) higher than the raw product pressure. Note the differential pressure at the point the light turns off. …..

Page 289

TEST 10.

MILK or milk PRODUCT-FLOW CONTROLS AND MILK or milk PRODUCT TEMPERATURE AT CUT-IN AND CUT-OUT

References:  Item 16p (B), (C) and (ED) …..

10.1 HTST PASTEURIZERS

Frequency:  Upon installation; at least once each three (3) months thereafter by the Regulatory Agency, or HACCP qualified industry person, acceptable to the Regulatory Agency, qualified under Item 16p(ED)2; daily by the milk plant operator; or when a regulatory seal has been broken. …..

Page 290

10.2 PASTEURIZERS AND ASEPTIC PROCESSING SYSTEMS USING INDIRECT HEATING

Application:  All HHST and HTST pasteurizers pasteurization systems with the FDD located downstream of the regenerator and/or final cooler and aseptic processing systems using indirect heating. When testing aseptic processing systems, the "milk or milk product divert system", or “milk or milk product divert valve" or "acceptable control system" may be substituted for the “FDD" when it is referenced in this Test. …..

Criteria: The pasteurizer or aseptic processor shall not operate in forward-flow unless the pasteurization or aseptic processing temperature has been achieved. The milk or milk product flow shall be diverted at a temperature lower than the chosen pasteurization or aseptic processing standard. …..

Procedure: …..

2.  	After the cut-in temperature has been determined and while the bath is above the cut-in temperature, allow the bath to cool slowly at a rate not exceeding 0.5ºC (1ºF) per thirty (30) seconds. Observe the temperature reading on the thermal-limit-controller when the test lamp goes out, cut-out temperature.  Determine that the cut-out temperature, on the thermal-limit-controller is equivalent to or greater than the chosen pasteurization or aseptic processing standard. Where adjustment is necessary, refer to the manufacturer's instructions.  After adjustment, repeat the procedure above, and when the results are satisfactory, record the results for the office records. …..

Page 291

10.3 PASTEURIZERS AND ASEPTIC PROCESSING SYSTEMS USING DIRECT HEATING

Application: All HHST and HTST pasteurizers pasteurization systems with the FDD located downstream of the regenerator and/or final cooler and aseptic processing systems using direct heating. When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. …..

.

Criteria: The pasteurizer or aseptic processor shall not operate in forward-flow unless the pasteurization or aseptic processing temperature has been achieved. The milk or milk product flow shall be diverted at a temperature lower than the chosen pasteurization or aseptic processing standard. …..

Procedure: …..

2.  	After the cut-in temperature has been determined and while the bath is above the cut-in temperature, allow the bath to cool slowly at a rate not exceeding 0.5ºC (1ºF) per thirty (30) seconds.  Observe the temperature reading on the thermal-limit-controller when the test lamp goes out, cut-out temperature.  Determine that the cut-out temperature, on the thermal-limit- controller, is equivalent to or greater than the chose pasteurization or aseptic processing standard.  Where adjustment is necessary, refer to the manufacturer's instructions.  After adjustment, repeat the procedure above and when the results are satisfactory, record the results for the office record. …..

TEST 11.

CONTINUOUS-FLOW HOLDING TUBES - HOLDING TIME

Reference:  Item 16p (B), (C) and (ED) …..

Page 295

11.2A MAGNETIC FLOW METER BASED TIMING SYSTEMS CONTINUOUS-FLOW  HOLDING TIME

TEST OPTION I

Note: The appropriate temperature elements may be placed in a water or oil bath to simulate the normal pasteurization or aseptic processing temperature of the holding tube as an alternative to heating the water in the system above the pasteurization or aseptic processing temperature.

Page 296

11.2B CONTINUOUS-FLOW HOLDING TUBES - FLOW ALARM

Application:  To all continuous-flow pasteurization and aseptic processing systems using a magnetic flow meter based timing system to replace a timing pump. When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. …..

Criteria:  When flow rate equals or exceeds the value at which the holding time was measured, the flow alarm shall cause the FDD to assume the diverted position, even though the temperature of the milk or milk product in the holding tube is above the pasteurization or aseptic processing temperature. …..

Procedure:

1. 	Operate the pasteurizer or aseptic processing equipment in forward-flow, below the high flow alarm, using water above the pasteurization or aseptic processing temperature.

NOTE: The appropriate temperature elements may be placed in a water or oil bath to simulate the normal pasteurization or aseptic processing temperature of the holding tube as an alternative to heating the water in the system above the pasteurization or aseptic processing temperature.  Observation and recording of this temperature should be done as described in Procedures 3 and 4 below. …..

Page 297

11.2C CONTINUOUS-FLOW HOLDING TUBES - LOW FLOW/LOSS-OF-SIGNAL ALARM

Application:  To all continuous-flow pasteurization and aseptic processing systems using a magnetic flow meter based timing system to replace a timing pump. When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. …..

Procedure:

1.  	Operate the pasteurizer or aseptic processing equipment in forward-flow, at a flow rate below the flow alarm set point and above the low flow/loss-of-signal alarm set point, using water. …..

Page 299

11.2F  HIGH FLOW ALARM RESPONSE TIME

Application:  To all continuous-flow pasteurization and aseptic processing systems using a magnetic flow meter based timing system to replace a timing pump. When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. …..

Procedure:

1.  Operate the pasteurizer or aseptic processing equipment in forward-flow, at a flow rate 25% below the high flow alarm as determined in Test 11.2B (Procedure 2).

2.   Mark the recorder chart with the high flow alarm set point.

NOTE: The appropriate temperature elements may be placed in a water or oil bath to simulate the normal pasteurization or aseptic processing temperature of the holding tube as an alternative to heating the water in the system above the pasteurization or aseptic processing temperature.  Observation and recording of this temperature should be done as described in Procedures 3 and 4 below. …..

Page 305

TEST 12.

THERMAL-LIMIT-CONTROLLER FOR CONTROL - SEQUENCE LOGIC

References:  Items 16p(B) and (ED)

Thermal-limit-controllers used with HHST and HTST pasteurizers pasteurization systems that have the FDD located downstream from the regenerator and/or cooler and aseptic processing systems shall be tested by one (1) of the following applicable Tests at the frequency prescribed:

Page 306

12.1 PASTEURIZATION AND ASEPTIC PROCESSING - INDIRECT HEATING

Application: To all HHST and HTST pasteurizers pasteurization systems that have the FDD located downstream from the regenerator and/or cooler and aseptic processing systems using indirect heating. When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. …..

Criteria:  The pasteurizer, or aseptic processing equipment, shall not operate in forward-flow until the milk or milk product surfaces downstream from the holding tube have been sanitized, or in the case of aseptic processing equipment, sterilized.  Upon start-up, surfaces shall be exposed to fluid at pasteurization temperature, or in the case of aseptic processing equipment, sterilizing temperature, for at least the required pasteurization or sterilization time.  If any public health control causes the FDD to assume the diverted flow position due to incorrect temperature, pressure or flow, forward-flow shall not be re-achieved until the milk or milk product-contact surfaces downstream from the holding tube have been re-sanitized, or in the case of aseptic processing equipment, re-sterilized. …..

Procedure:  …..

3.  	Immerse the sensing element from the holding tube in the bath.  The test lamp should light up, i.e., forward-flow after a minimum time delay of one (1) second for continuous-flow pasteurization systems. For aseptic processing systems no delay is required if the filed process includes a documented sterilization period. …..

Page 307

12.2 PASTEURIZATION AND ASEPTIC PROCESSING - DIRECT HEATING

Application: To all HHST and HTST pasteurizers pasteurization systems that have the FDD located downstream from the regenerator and/or cooler and aseptic processing systems using direct contact heating. When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. …..

Criteria:  The pasteurizer, or aseptic processing equipment, shall not operate in forward-flow until the milk or milk product surfaces downstream from the holding tube have been sanitized, or in the case of aseptic processing equipment, sterilized.  Upon start-up, surfaces shall be exposed to fluid at pasteurization temperature, or in the case of aseptic processing equipment, sterilizing temperature for at least the required pasteurization or sterilization time.  If the milk or milk product temperature falls below the pasteurization or sterilization standard in the holding tube, forward-flow shall not be re-achieved until the milk or milk product-contact surfaces downstream from the holding tube have been re-sanitized, or in the case of aseptic processing equipment, re-sterilized. …..

Procedure: …..

5.  	Immerse the third sensing element located at the holding tube, into the bath. The test lamp should light up, i.e., forward-flow, after a minimum time delay of one (1) second for continuous- flow pasteurization systems. For aseptic processing systems no delay is required if the filed process includes a documented sterilization period. …..

Page 308

TEST 13.

SETTING OF CONTROL SWITCHES FOR milk or milk PRODUCT

PRESSURE IN THE HOLDING TUBE

Reference:   Item 16p(B) and (ED)

Application: To all HHST pasteurizers and aseptic processing pasteurization systems, which are capable of operating with product in forward-flow mode, with less than 518 kPa (75 psig) pressure in the holding tube.  When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. …..

Criteria: The pasteurizer or aseptic processor shall not operate in forward-flow unless the product pressure in the holding tube is at least 69 kPa (10 psi) above the boiling pressure of the product. …..

Procedure: …..

Page 309

For each HHST pasteurizer or aseptic processing system temperature, the milk or milk product pressure switch setting is as follows: …..

TEST 14.

SETTING OF CONTROL SWITCHES FOR DIFFERENTIAL

PRESSURE ACROSS THE INJECTOR

Reference: Item 16p(B) and (ED)

Application: To all continuous flow pasteurizers pasteurization systems and aseptic processing systems using direct injection heating. When testing aseptic processing systems, the "milk or milk product divert system" or "milk or milk product divert valve" or "acceptable control system" may be substituted for the "FDD" when it is referenced in this Test. …..

Criteria: The pasteurizer or aseptic processor shall not operate in forward-flow unless the milk or milk product pressure drop across the injector is at least 69 kPa (10 psi). …..

Page 310

TEST 15.

ELECTRO-MAGNETIC INTERFERENCE FROM HAND-HELD

COMMUNICATION DEVICES

Application: To all electronic controls devices used to assure compliance with public health safeguards on continuous flow pasteurization and aseptic processing equipment that are installed in milk plants. …..

Page 311

Procedure: …..

5.  	Repeat the Test for each electronic control device used to regulate a pasteurization or aseptic processing system’s public health safeguard(s).

For Example: For the temperature set point, operate the pasteurization or aseptic processing equipment on water in diverted-flow in the “Product” mode, at a steady temperature within 3ºC (5ºF) of the lowest cut-in temperature. In this example, an adverse effect is defined as the forward-flow movement of the FDD or any artificial increase in temperature. …..

Make the following changes to APPENDIX K. HACCP PROGRAM on Page 328:

II. IMPLEMENTATION OF A HACCP SYSTEM

Page 328

Verification and Validation:

1.	Verification: Every milk plant, receiving station or transfer station shall verify that the HACCP System is being implemented according to design, except that critical factors for aseptically processed Grade “A” milk and milk products, as determined by the process authority and listed on the scheduled process under 21 CFR Part 113 the APPS, as defined by this Ordinance, shall be managed separately from the NCIMS HACCP System, even if identified as a CCP in the hazard analysis. Critical factors shall be monitored under the operating supervision of an individual who has successfully completed an approved course of instruction in low-acid canned foods as required under 21 CFR 108.35.  Compliance with the provisions of 21 CFR Part 113 shall satisfy the requirements of this Section, regardless of whether a critical factor has also been designated as a CCP.  The milk plant's APPS shall be inspected by FDA, or the State Regulatory Agency when designated by FDA, in accordance with the applicable requirements of 21 CFR Parts 108, 110 and 113  at a frequency determined by FDA. …..

Make the following change to APPENDIX L. APPLICABLE REGULATIONS, STANDARDS OF IDENTITY FOR MILK AND MILK PRODUCTS AND THE FEDERAL FOOD, DRUG, AND COSMETIC ACT on Page 335:

Page 335

21 CFR PART 113 - THERMALLY PROCESSED LOW-ACID FOODS PACKAGED IN HERMETICALLY SEALED CONTAINERS

21 CFR PART 114 – ACIDIFIED FOODS

21 CFR 130.10 - Requirements for foods named by use of a nutrient content claim and a standardized term …..

Page 337

Within the Forms cited in APPENDIX M-REPORTS AND RECORDS on page 337, the following changes to FORM FDA 2359-MILK PLANT INSPECTION REPORT and FORM FDA 2359b-MILK PLANT EQUIPMENT TEST REPORT shall be made.

| Department of Health and Human Services Public Health Service Food and Drug Administration |  | MILK PLANT INSPECTION REPORT (Includes Dry Milk/Condensing Plants, Receiving Stations, Transfer Stations, and Milk Tank Truck Cleaning Facilities) |  |  |  | INSPECTING AGENCY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAME AND LOCATION OF PLANT |  |  |  | Pounds Sold Daily Milk |  |  |  |  |  |  |  |
|  |  |  |  | Other Milk Products |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |
|  |  |  |  | Permit No. |  |  |  |  |  |  |  |
| Inspection of your plant today showed violations existing in the Items checked below. You are further notified that this inspection report serves as notification of the intent to suspend your permit if the violations noted are not in compliance at the time of the next inspection. (Refer to Sections 3 and 5 of the Grade “A” Pasteurized Milk Ordinance.) |  |  |  |  |  |  |  |  |  |  |  |
| 1. FLOORS: Smooth; impervious; no pools; good repair; trapped drains (a) 2. WALLS AND CEILINGS: Smooth; washable; light-colored; good repair (a) 3. DOORS AND WINDOWS: All outer openings effectively protected against entry of flies and rodents (a) Outer doors self-closing; screen doors open outward (b) 4. LIGHTING AND VENTILATION: Adequate light in all rooms (a) Well ventilated to preclude odors and condensation; filtered air with pressure systems (b) 5. SEPARATE ROOMS: Separate rooms as required; adequate size (a) No direct opening to barn or living quarters (b) Storage tanks properly vented (c) 6. TOILET FACILITIES: Complies with local Ordinances (a) No direct opening to processing rooms; self-closing doors (b) Clean; well-lighted and ventilated; proper facilities (c) Sewage and other liquid wastes disposed of in sanitary manner (d) 7. WATER SUPPLY: Constructed and operated in accordance with Ordinance (a) No direct or indirect connection between safe and unsafe water (b) Condensing water and vacuum water in compliance with Ordinance requirements (c) Reclaim water complies with Ordinance (d) Complies with bacteriological standards (e) 8. HANDWASHING FACILITIES: Located and equipped as required; clean and in good repair; improper facilities not used (a) 9. MILK PLANT CLEANLINESS: Neat; clean; no evidence of insects or rodents; trash properly handled (a) No unnecessary equipment (b) 10. SANITARY PIPING: Smooth; impervious, corrosion-resistant, non-toxic, easily cleanable materials; good repair; accessible for inspection (a) Mechanically cleaned lines meet Ordinance specs (b) Pasteurized products conducted in sanitary piping, except as permitted by Ordinance (c) 11. CONSTRUCTION AND REPAIR OF CONTAINERS AND EQUIPMENT: Smooth, impervious, corrosion-resistant, non-toxic, easily cleanable materials; good repair; accessible for Inspection (a) Self-draining; strainers of approved design (b) Approved single-service articles; not reused (c) 12. CLEANING AND SANITIZING OF CONTAINERS/ EQUIPMENT: Containers, utensils, and equipment effectively cleaned (a) Mechanical cleaning requirements of Ordinance in Compliance; records complete (b) Approved sanitization process applied prior to use of product-contact surfaces (c) Required efficiency tests in compliance (d) Multi-use plastic containers in compliance (e) |  |  | Aseptic system sterilized (f) 13. STORAGE OF CLEANED CONTAINERS AND EQUIPMENT: Stored to assure drainage and protected from contamination (a) 14. STORAGE OF SINGLE-SERVICE ARTICLES: Received, stored and handled in a sanitary manner; paperboard containers not reused except as permitted by the Ordinance (a) 15a. PROTECTION FROM CONTAMINATION: Operations conducted and located so as to preclude contamination of milk, milk products, ingredients, containers, equipment, and utensils (a) Air and steam used to process products in compliance with Ordinance (b) Approved pesticides, safely used (c) 15b. CROSS CONNECTIONS: No direct connections between pasteurized and raw milk or milk products (a) Overflow, spilled and leaked products or ingredients discarded (b) No direct connections between milk or milk products and cleaning and/or sanitizing solutions (c) 16a. PASTEURIZATION-BATCH: (1) INDICATING AND RECORDING THERMOMETERS: Comply with Ordinance Specifications (a) (2) TIME AND TEMPERATURE CONTROLS : Adequate agitation throughout holding; agitator sufficiently submerged (a) Each pasteurizer equipped with indicating and recording thermometer; bulb submerged (b) Recording thermometer reads no higher than indicating thermometer(c) Product held minimum pasteurization temperature continuously for 30 minutes, plus filling time if product preheated before entering vat, plus emptying time, if cooling is begun after opening outlet (d) No product added after holding begun (e) Airspace above product maintained at not less than 5.0° F higher than minimum required pasteurization temperature during holding (f) Approved airspace thermometer; bulb not less than 1 inch above product level (g) Inlet and outlet valves and connections in compliance with Ordinance (h) 16b. PASTEURIZATION-HIGH TEMPERATURE: (1) INDICATING AND RECORDING THERMOMETERS: Comply with Ordinance specifications (a) (2) TIME AND TEMPERATURE CONTROLS: Flow-diversion device complies with Ordinance requirements (a) Recorder controller complies with Ordinance requirements (b) Holding tube complies with Ordinance requirements (c) Flow promoting devices comply with Ordinance requirements (d) (3) ADULTERATION CONTROLS: Satisfactory means to prevent adulteration with added water (a) 16c. ASEPTIC PROCESSING: (1) INDICATING AND RECORDING THERMOMETERS: Comply with Ordinance Specifications (a) |  | (2) TIME AND TEMPERATURE CONTROLS: Flow-diversion device complies with Ordinance requirements (a) Recorder controller complies with Ordinance requirements (b) Holding tube complies with Ordinance requirements (c) Flow promoting devices comply with Ordinance requirements (d) (3) ADULTERATION CONTROLS: Satisfactory means to prevent adulteration with added water (a) 16dc. REGENERATIVE HEATING: Pasteurized or aseptic product in regenerator automati- cally under greater pressure than raw product in regenerator at all times (a) Accurate pressure gauges installed as required; booster pump properly identified and installed (b) Regenerator pressures meet Ordinance Requirements (c) 16ed. RECORDING CHARTS: Batch pasteurizer charts comply with applicable Ordinance Requirements (a) HTST and HHST pasteurizer charts comply with applicable Ordinance Requirements (b) Aseptic charts comply with applicable Ordinance Requirements (c) 17. COOLING OF MILK: Raw milk maintained at 45° F or less until processed (a) Pasteurized milk and milk products, except those to be cultured, cooled immediately to 45° F or less in approved equipment; all milk and milk products stored there until delivered (b) Approved thermometer properly located in all refrigeration rooms and storage tanks (c) Recirculated cooling water from safe source and properly protected; complies with bacteriological standards (d) 18. BOTTLING AND PACKAGING: Performed in a plant where contents finally pasteurized (a) Performed in a sanitary manner by approved mechanical equipment (b) Aseptic filling in compliance (c) 19. CAPPING: Capping and/or closing performed in sanitary manner by approved mechanical Equipment (a) Imperfectly capped/closed products properly handled (b) Caps and/or closures comply with Ordinance (c) 20. PERSONNEL CLEANLINESS: Hands washed clean before performing plant functions; rewashed when contaminated (a) Clean outer garments and hair covering worn (b) No use of tobacco in processing areas (c) 21. VEHICLES: Vehicles clean; constructed to protect milk (a) No contaminating substances transported (b) 22. SURROUNDINGS: Neat and clean; free of pooled water, harborages, and breeding areas (a) Tank unloading areas properly constructed (b) Approved pesticides, used properly (c) |  |  |  |  |  |  |
| REMARKS |  |  |  |  |  |  |  |  |  |  |  |
| 1. A receiving station shall comply with Items 1 to 15, inclusive, and 17, 20, and 22. Separation requirements of Item 5 do not apply. 2. A transfer station shall comply with Items 1, 4, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 20, 22 and as climatic and operating conditions require, applicable provisions of Items 2 and 3. In every case, overhead protection shall be required. 3. Facilities for the cleaning and sanitizing of milk tank trucks shall comply with the same requirements for transfer stations. 4. In areas of the milk plant where Items 7, 10, 11, 12, 13, 15, 17, 18 and 19 are dedicated only to the Aseptic Processing and Packaging System, as defined by the PMO, these Items shall be inspected and regulated in accordance with the applicable requirements of 21 CFR Parts 108,110 and113. |  |  |  |  |  |  |  |  |  |  |  |
| DATE | SANITARIAN |  |  |  |  |  |  |  |  |  |  |
| NOTE – Item numbers correspond to required sanitation Items for Grade “A” pasteurized milk in the Grade “A” Pasteurized Milk Ordinance. |  |  |  |  |  |  |  |  |  |  |  |

FORM FDA 2359 (10/08 10/12)

| DEPARTMENT OF HEALTH AND HUMAN SERVICES FOOD AND DRUG ADMINISTRATION |  |  |  |  | MILK PLANT EQUIPMENT TEST REPORT |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TEST NO. | TEST |  |  |  |  | TEST FREQUENCY | TESTED (X or NA) | RESULTS OF TEST (See Reverse for Working Notes) |  |
| 1. | Indicating Thermometers (including air space): Temperature Accuracy |  |  |  |  | 3 months |  |  |  |
| 2. | Recording Thermometers: Temperature Accuracy |  |  |  |  | 3 months |  |  |  |
| 3. | Recording Thermometers: Time Accuracy |  |  |  |  | 3 months |  |  |  |
| 4. | Recording Thermometers: Checked against Indicating Thermometer |  |  |  |  | 3 months |  | Daily by operator |  |
| 5. | Flow-Diversion Device (FDD): Proper Assembly and Function (HTST and HHST) |  |  |  |  |  |  |  |  |
|  | 5.1 | Leakage Past Valve Seat(s) |  |  |  | 3 months |  |  |  |
|  | 5.2 | Operation of Valve Stem(s) |  |  |  | 3 months |  |  |  |
|  | 5.3 | Device Assembly (micro-switch) Single Stem |  |  |  | 3 months |  |  |  |
|  | 5.4 | Device Assembly (micro-switches) Dual Stem |  |  |  | 3 months |  |  |  |
|  | 5.5 | Manual Diversion - Parts (A, B, and C) (HTST only) |  |  |  | 3 months |  |  |  |
|  | 5.6 | Response Time |  |  |  | 3 months |  |  |  |
|  | 5.7 | Time Delay Interlock (dual stem devices) (Inspect) |  |  |  | 3 months |  |  |  |
|  | 5.8 | Time Delay Interlock (dual stem devices) (CIP) |  |  |  | 3 months |  |  |  |
|  | 5.9 | Leak Detect Flush Time Delay (HTST only as applicable) |  |  |  | 3 months |  |  |  |
| 6. | Leak-Protect Valves: Leakage (Vats only) |  |  |  |  | 3 months |  |  |  |
| 7. | Indicating Thermometers on Pipelines: Thermometric Response (HTST only) |  |  |  |  | 3 months |  |  |  |
| 8. | Recorder-Controller: Thermometric Response (HTST only) |  |  |  |  | 3 months |  |  |  |
| 9. | Regenerator Pressure Controls |  |  |  |  |  |  |  |  |
|  | 9.1 | Pressure Switches (HTST only) |  |  |  | 3 months |  |  |  |
|  | 9.2 | Differential Pressure Controllers |  |  |  |  |  |  |  |
|  | 9.2.1 | Calibration |  |  |  | 3 months |  |  |  |
|  | 9.2.2 | Interwiring Booster Pump (HTST only) |  |  |  | 3 months |  |  |  |
|  | 9.2.3 | Interwiring FDD (HTST*, and HHST and Aseptic) |  |  |  | 3 months |  |  |  |
|  | 9.3 | Additional Booster Pump Interwiring (HTST only) |  |  |  |  |  |  |  |
|  | 9.3.1 | With FDD |  |  |  | 3 months |  |  |  |
|  | 9.3.2 | With Metering Pump |  |  |  | 3 months |  |  |  |
| 10. | Milk-Flow Controls: Cut-in and Cut-out Temperatures (10.1, 10.2* or 10.3*) |  |  |  |  | 3 months |  | Daily by operator (HTST) |  |
| 11. | Timing System Controls |  |  |  |  |  |  |  |  |
|  | 11.1 | Holding time (HTST, except Magnetic Flow Meters) |  |  |  | 6 months |  | Adjusted product holding time if applicable |  |
|  | 11.2.a | Magnetic Flow Meters (HTST only) |  |  |  | 6 months |  |  |  |
|  | 11.2.b | Flow Alarm (HTST, and HHST and Aseptic) |  |  |  | 6 months |  |  |  |
|  | 11.2.c | Loss of Signal Alarm (HTST, and HHST and Aseptic) |  |  |  | 6 months |  |  |  |
|  | 11.2.d | Flow Cut-in/Cut-out (HTST only) |  |  |  | 6 months |  |  |  |
|  | 11.2.e | Time Delay (after divert) (HTST with a FDD located at the end of the holding tube) |  |  |  | 6 months |  |  |  |
|  | 11.2.f | High Flow Alarm Response Time (All Magnetic Flow Meter Systems) |  |  |  | 6 months |  |  |  |
|  | 11.3 | HHST Indirect Heating |  |  |  | 6 months |  |  |  |
|  | 11.4 | HHST Direct Injection Heating |  |  |  | 6 months |  |  |  |
|  | 11.5 | HHST Direct Infusion Heating |  |  |  | 6 months |  |  |  |
| 12. | Controller: Sequence Logic (HHST and Aseptic) (12.1* or 12.2*) |  |  |  |  | 3 months |  |  |  |
| 13. | Product Pressure-Control Switch Setting (HHST and Aseptic) |  |  |  |  | 3 months |  |  |  |
| 14. | Injector Differential Pressure Injection Heating (HTST*, and HHST and Aseptic) |  |  |  |  | 3 months |  |  |  |
| 15. | Electro-Magnetic Interference from Hand-Held Communication Devices (HTST, and HHST and Aseptic) |  |  |  |  | 3 months |  |  |  |
|  | *For HTST systems with the FDD located downstream of the regenerator and/or cooler section. |  |  |  |  |  |  |  |  |
| REMARKS (If additional space is required please place information on the back of this form or on a separate page.) |  |  |  |  |  |  |  |  |  |
| PLANT |  |  | IDENTITY OF EQUIPMENT | LOCATION |  |  |  | DATE | SANITARIAN |
| NOTE: This Form is a supplement to the Milk Plant Inspection Report, FORM FDA 2359, and these tests are in addition to the equipment requirements for which compliance is determined by inspection. (Refer to Appendix I of the Grade "A" Pasteurized Milk Ordinance.) |  |  |  |  |  |  |  |  |  |

FORM FDA 2359b (10/08 10/12)   (PREVIOUS EDITIONS ARE OBSOLETE)

Make the following change to Appendix Q.  Operation of Automatic Milking Installations for the Production of Grade “A” Raw Milk for Pasteurization on Page 356:

Page 356

Appendix Q.  Operation of Automatic Milking Installations for the Production of Grade “A” Raw Milk for Pasteurization, ULTRA-PASTEURIZATION OR ASEPTIC PROCESSING AND PACKAGING

This Appendix is intended to clarify how AMIs are to perform to be considered in compliance with the Grade "A" PMO. It is formatted to follow the Items as outlined in Section 7. STANDARDS FOR GRADE “A” RAW MILK FOR PASTEURIZATION, ULTRA-PASTEURIZATION OR ASEPTIC PROCESSING AND PACKAGING.  Both requirements and recommendations are discussed.

Make the following change to Appendix r.  determination of time/ temperature control for safety milk and milk products on Page 359:

Page 359

5. 	Is the milk or milk product processed and packaged so that it no longer requires TCS; such as, Grade “A” aseptically processed and packaged milk and milk products? …..

Add a new APPENDIX S. ASEPTIC PROCESSING AND PACKAGING PROGRAM on Page 362:

APPENDIX S. ASEPTIC PROCESSING AND PACKAGING PROGRAM

The Aseptic Processing and Packaging Program is designed to include all low-acid (21 CFR Part 113) Grade “A” aseptic processed and packaged milk and milk products.

Inspections of a milk plant or portion of a milk plant that is IMS listed to produce aseptically processed and packaged milk or milk products shall be conducted by the Regulatory Agency in accordance with this Ordinance and the information provided below at least once every six (6) months.  The APPS, as defined by this Ordinance, shall be exempt from Items 7p, 10p, 11p, 12p, 13p, 15p, 16p, 17p, 18p, and 19p of this Ordinance and shall comply with the applicable portions of 21 CFR Parts 108, 110 and 113.  The milk plant's APPS shall be inspected by FDA, or the State Regulatory Agency when designated by FDA, in accordance with the applicable requirements of 21 CFR Parts 108, 110 and 113 at a frequency determined by FDA.

When the APPS, as defined by this Ordinance, is utilized to produce aseptically processed and packaged milk or milk products and pasteurized and/or ultra-pasteurized milk and milk products, the APPS shall be inspected and tested by the Regulatory Agency in accordance with the requirements cited in Section 7 of this Ordinance.

Aseptic PROCESSING AND PACKAGING Program CFR/PMO

comparison Summary Reference

| PMO, Section 7 Items | Aseptic Program | Authority |
| --- | --- | --- |
| 1p. Floors – Construction | Floor drains are not required in storage rooms for aseptic processed and packaged milk or milk products. | PMO |
| 2p. Walls and Ceiling –Construction | Ceiling requirements are exempt in aseptically processed and packaged milk or milk products dry storage rooms. (Same as for dry milk or milk products.) | PMO |
| 3p. Doors and Windows | None | PMO |
| 4p. Lighting and Ventilation | None | PMO |
| 5p. Separate Rooms | Fabrication of containers and closures for aseptic processed and packaged milk and milk products within the APPS is exempt. | PMO |
| 6p. Toilet – Sewage Disposal Facilities | None | PMO |
| 7p. Water Supply* | The APPS is exempt, but shall comply with the CFR. | PMO/CFR |
| 8p. Handwashing Facilities | None | PMO |
| 9p. Milk Plant Cleanliness | None | PMO |
| 10p. Sanitary Piping* | The APPS is exempt, but shall comply with the CFR. | PMO/CFR |
| 11p. Construction and Repair of Containers and Equipment* | The APPS is exempt, but shall comply with the CFR. Paper, plastics, foil, adhesives and other components of containers and closures used in the packaging of milk or milk products that have been aseptically processed and packaged are not required to comply with Appendix J of the PMO,; originate from an IMS Listed Source,; and are subject to the requirements of the CFR. | PMO/CFR |
| 12p. Cleaning and Sanitizing of Containers and Equipment* | The APPS is exempt, but shall comply with the CFR. | PMO/CFR |
| 13p. Storage of Cleaned Containers and Equipment* | The APPS is exempt, but shall comply with the CFR. | PMO/CFR |
| 14p. Storage of Single- Service Containers, Utensils and Materials | None | PMO |
| 15p.(A) Protection from Contamination* | The APPS is exempt, but shall comply with the CFR. | PMO/CFR |
| 15p.(B) Protection from Contamination - Cross Connections* | The APPS is exempt, but shall comply with the CFR. APPS equipment is exempt from the separation requirements of the PMO in relationship to instrumented steam blocks between milk and milk products and cleaning and/or chemical sanitizing solutions. | PMO/CFR |
| 16p. Pasteurization and Aseptic Processing and Packaging (A) through (D)* | The APPS is exempt, but shall comply with the CFR. The State Regulatory Agency is not required to conduct the quarterly equipment testing and sealing of aseptic processing equipment. Records and recording charts are not required to be reviewed during routine inspections, State ratings or check ratings. | CFR |
| 17p. Cooling of Milk and Milk Products* | The APPS and aseptic processed and packaged product storage is exempt, but shall comply with the CFR. | PMO/CFR |
| 18p. Bottling, Packaging and Container Filling* | The APPS is exempt, but shall comply with the CFR. | CFR |
| 19p. Capping, Container Closure and Sealing and Dry Milk Product Storage* | The APPS is exempt, but shall comply with the CFR. | CFR |
| 20p. Personnel -Cleanliness | None | PMO |
| 21p. Vehicles | None | PMO |
| 22p. Surroundings | None | PMO |

* Note: In areas of the milk plant where these Items are dedicated only to the APPS, as defined by this Ordinance, these Items shall be inspected and regulated in accordance with the applicable FDA regulations (21 CFR Parts 108, 110 and 113).

Make the following change to the INDEX on Pages 362-379:

Page 362

Aseptic

aseptically processed processing and packaging, definition

aseptic processing and packaging systems system, definition

controlling added water

cooling exemption

examination of

imminent hazard

labeling

process authority

resterilized after contamination

sampler-milk tank truck…………………………………………………………………

Page 368

Flow diversion device	………………………………………………………………………

aseptic systems

leak escape

Page 369

Indicating thermometer

airspace…………………………………………………………………………………

aseptic system…………………………………………………………………………..

Page 370

Industry

aseptic processing records

certified farm inspection

records for aseptically packaged products

records, drug residue testing

Low-acid aseptic milk and milk products, definition………………………………………

Page 371

Milk

acidified, definition

adulterated

aseptically processed processing and packaging, definition

flavored

low-acid aseptic, definition…………………………………………………………….

lowfat

Page 372

Packaging

aseptic…………………………………………………………………………………

equipment……………………………………………………………………………..

Page 373

Pasteurization, definition…………………………………………………………………

milk or milk product-to-water-to-milk or milk product regenerative heating

pasteurization and aseptic processing records, equipment tests and examination

pressure relief valves……………………………………………………………………

Page 374

Recording thermometer……………………………………………………………………

aseptic system

aseptic system, chart reviewed by management

batch pasteurizer………………………………………………………………………..

Page 375

Records

access to industry

aseptic processing

batch pasteurization…………………………………………………………………….

Page 377

Storage

aseptically processed and packaged milk

Page 378

Temperature

aseptic milk storage

charts……………………………………………………………………………………

Tests

airspace thermometer

aseptic processing equipment

aseptically packaged milk and milk products

booster pump……………………………………………………………………………

Page 379

Thermometer

airspace

aseptic system

batch pasteurization…………………………………………………………………….